CENWP-OD-B                                                                                                                                  12 January 2009
MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist
SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 2 of 2009, which covers the period from 04 through 10 January 2009.
1 OPERATION SUMMARY

1.1 Daily average river flows ranged from 173.4 to 207.5 kcfs.  Daily average project tailwater ranged from 17.0’ to 20.7’ msl.  Daily average powerhouse forebay elevation ranged from 72.5’ to 74.9’ msl.  Daily average river temperature ranged from 38ºF to 40ºF.  Secchi disk measurements were between 2.5’ and 4.5’.
1.2 Spill Bay 1, which provides attraction flow for Cascades Island, is open 6” during daylight hours.  Daily average spill ranged from 0.5 to 1.0 kcfs.  
1.3 Unit Operation.  Powerhouse 1 became the priority powerhouse on 10 January.
1.3.1 Unit 1 was out of service from 1104 until 1401 on 7 January for STS removal.

1.3.2 Unit 3 was out o f service from 0734 until 1102 on 7 January for STS removal.

1.3.3 Unit 7 has remained out of service since 04 June 2007 for turbine rehab.

1.3.4 Unit 8 has remained out of service since 17 December due to turbine bearing failure.
1.3.5 Unit 9 has remained out of service since 04 August for turbine rehab.
1.3.6 Unit 10 was out of service from 0019 until 0818 on 5 January due to a hot bearing.  It was out of service again from 1507 on 5 January until 0410 on 6 January due to a babbitt found in the turbine bearing strainer.  At 0418 on 6 January until 1337 on 7 January, it was OOS due to drawdown levels.
1.3.7 Unit 11 has remained out of service since 12 June due to rotor cracks.
1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.
1.4.1 F1 has remained out of service since 01 December for annual maintenance.

1.4.2 F2 was on standby from 0936 until 1141 on 8 January to facilitate F1 testing.
1.5 Gatewell drawdowns are measured on Wednesdays.
2 ACTIVITIES

2.1 Auxiliary Water System Closures.  The following AWS valves were closed this reporting week:
2.1.1 FV 1-1, FV3-7, FV3-9, FV4-3 and FV4-4 returned to service at 1115 on 9 January.
2.1.2 FV3-7 was closed for cleaning from 1110 until 1145 from 10 January and closed to float trash from 2209 on 10 January until 0500 on 11 January.
2.1.3 FV6-9 was closed from 1019 until 1159 on 5 January for cleaning and was in manual from 0030 until 1416 on 8 January due to debris on the trashrack.  FV6-9 was closed at 0802 on 10 January to facilitate the WS dewater activities.
2.1.4 Weir 37 was closed from 1130 until 1640 on 5 January and from 0130 until 1416 on 8 January to float trash.
2.2 STS/VBS inspections.  The following STS/VBS removal activities occurred this reporting week.  
Table 1.  STS and VBS inspections.  
	Unit
	Date
	Run Hours
	Comments

	1, 3
	1/7
	NA
	Removed until 15 September.

	11-18
	NA
	NA
	Six STSs have broken gearboxes due to freezing in December.


2.3 Dewatering and Fish Salvage.  The following fish salvage and dewatering activities occurred this week:
2.3.1 On 05 January, biologists removed two juvenile chinook, one juvenile steelhead, one peamouth, and 25 suckers from the AFF.  Fish were released back to the WS ladder.  One large sturgeon was moved to the AFF entrance bulkhead, with the hopes it would move into the WS ladder.  Biologists also removed were three dead and fungus covered adult salmonids from the brail pool.
3 RESEARCH  
3.1 WDFW Fish Counting.  Daily video counting at Washington Shore concluded on 09 January.  Video counting at Cascades Island and Bradford Island began on 10 January.
3.2 WDFW/ODFW/CRITFC Sea lion hazing.  Hazing from boats began on 06 January.

4 FISHWAYS 

4.1 Project biologists inspected 04, 06-09 January.
4.2  Fish Passage Plan Observations.
Table 2.  FPP Out of Criteria items.

	DATE
	LOCATion
	FPP itEMS
	CAUSE
	RESPONSE
	TR#

	BRADFORD ISLAND – returned to service at 1115 on 9 January

	CASCADES ISLAND 

	1/6-7
	CI entrance
	Head <1.0’>
	System in manual
	Operators adjusted.
	

	1/7-9
	CI diffusers
	FG6-9 open, FG6-10 closed.
	Compensate for FG6-10/12
	No action.
	

	1/7-9
	CI diffusers
	FG6-12 closed
	No power
	Operators TR’d
	

	1/6
	CI ent. gates
	S. gate open
	System in manual
	Operators adjusted.
	

	
	
	
	
	
	

	WASHINGTON SHORE- went to orifice flow on 10 January.

	1/7-8
	Weir 67
	Reading <1.0’
	Trash on FV6-9
	Riggers cleaned.
	

	1/8
	Weir 37 & 38
	Reading <1.0’
	Trash on FV6-9
	Riggers cleaned.
	

	1/7-9
	FV6-9 trashrack
	Head >0.5’
	Excessive debris
	Riggers cleaned.
	

	1/8
	F2
	Out of service
	F1 testing.
	Please see 1.4.2
	

	All week
	S. monolith ent.
	Gate<13’ open
	System in manual.
	Please see 4.5.6
	

	DSM1/ITS

	1/4
	ITS chain gate
	7A closed
	Forgot.
	Gate opened on 1/5
	

	DSM2/ JMF – OOS for winter maintenance


4.2.1 FG6-10 closed.  The motor was repaired on 15 August 2007.  The shaft and gate have separated.  The diffuser will not be operational until repaired in winter 08/09.  It is OOC when TW is above 19.0’ msl.
4.2.2 PH2 velocity meter.  The velocity meter is broken.  It will be replaced with a mechanical meter.

4.3 Fishway Temperatures.  Temperatures are taken 01 March through 30 November
4.4 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.  

4.5 Adult Fishways. 
4.5.1 Cascades Island fishway PLC is in manual.  Calibration was checked on 08 January.
4.5.2 DSM1 STSs were pulled on 07 January.  Dewatering is scheduled for 15 January.

4.5.3 BI fishway exit was opened at 1012 on 09 January.  WG 2 remains in manual due to an inability to raise >10.9’.  All other weir gates and fish valves returned to auto at 1115 on 9 January.
4.5.4 Cascades Island exit remains open.  The north UMT bulkhead was installed at 0755 and the picket leads were raised at 0823 on 9 January.
4.5.5 Due to a broken lifting beam, sea lion exclusion bars remain installed in the four large FOGs.  Stab plates remain installed in the smaller FOGs.  
4.5.6 Since 23 October 2008, the south monolith entrances (SUE and SDE) have been without control power and have been adjusted manually.  At this time, SUE and SDE are set at 9’ and 10’ respectively.  While the entrance opening is out of criteria, the entrance differential has been maintained around 1.0’ – 1.1’.  

4.5.7 The WS fishway went to orifice flow at 0802 on 10 January.  It will be dewatered to tailwater on 12 January.

4.5.8 The AFF was dewatered on 05 January.  

4.6 Juvenile Fishways.

4.6.1 DSM2 remains dewatered.  The two-mile pipe is scheduled to be inspected on 15 January.
4.6.2 The B2CC was opened from 0802 until 1005, with flushing water continuing until 1200, on 9 January.  This was to flush debris from the PH2 forebay.
4.7 Lamprey Fishways.  BI and WS AWS lamprey ramps are winterized.
5 CONSTRUCTION
5.1 PH1 Ice and Trash Sluiceway Wall Removal.  Automated gate installation in FY09.  Wall removal in FY10.
5.2 TIE crane.  The TIE crane boom repair contract has been awarded.  Completion expected in Spring ‘09.
6 HAZMAT, SPILLS AND CLEANUP. 
6.1 On 07 January at 0210 hrs, a barge passing downstream through the navigation lock reported what appeared to be a sheen of some type in the lock chamber. After notifying all appropriate state and federal agencies, Bonneville Dam held the barge in the navigation lock until Coast Guard and Corps personnel could inspect the vessel and determine the nature of the sheen.  Upon filling the navigation chamber to raise the vessel for inspection and examining the chamber in daylight, the sheen was no longer apparent and there was no damage to the hull of the vessel.  The Coast Guard cleared the barge to depart the navigation lock at 0900 hrs.
7 TELETYPES AND OTHER NOTEWORTHY ITEMS
7.1 A teletype sent on 06 November establishes the upper BON forebay limit as 76.5’ msl. and the lower limit as 71.5’ msl.  

7.2 A teletype sent on 31 December, regarding BON tailwater operations for chum egg incubation, requests a minimum tailwater elevation of 11.5’ msl during all hours from 0001 on 1 January 2009 until further notice. 
7.3 A teletype sent on 07 January requests a soft forebay constraint of 74.5’ msl or lower from 0600 until 1800 on 11 January.  This operation is in support of water flow sampling on Bradford Island.

7.4 A teletype sent on 9 January requests the B2Cc be opened from 0800-1000 to clear debris from the PH2 forebay.
7.5 Winter maintenance continues.  For a complete list of dewatering activities, please contact Project Fisheries.

JAMES R. MAHAR, P.E.

Operations Manager
1

