CENWP-OD-B                                                                                                                                  20 April 2009
MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist
SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 16 of 2009, which covers the period from 12 through 18 April 2009.
1 OPERATION SUMMARY

1.1 Daily average river flows ranged from 227.4 to 265.0 kcfs.  Daily average project tailwater ranged from 19.1’ to 20.8’ msl.  Daily average powerhouse forebay elevation ranged from 73.3’ to 74.8’ msl.  Daily average river temperature ranged from 47ºF to 49ºF.  Secchi disk measurements ranged from 2.5’ to 5.0’.
1.2 Spill Bays 1 and 18, which provide attraction flow for Cascades Island and B-Branch, respectively, are open 6” during daylight hours.  Daily average spill ranged from 99.4 to 100.9 kcfs.
1.2.1 Bay 17 remains set at two dogs.  Since Bay 17 is providing less flow at two dogs than at 3.5’, flow is compensated through Bay 7 and 13 by increasing the opening from 2’ to 2.5’.  This will be how BON is operated for spring spill to satisfy the 100K spill requirement
1.3 Unit Operation.  Powerhouse 2 remains the priority powerhouse.
1.3.1 Unit 5 was out of service (OOS) from 0015 until 0036 on 13 April, and from 1513 until 1551 on 15 April for Bank 7/8 switching.
1.3.2 Unit 6 was OOS from 0015 until 0036 on 13 April, and from 1514 until 1554 on 15 April for Bank 7/8 switching.

1.3.3 Unit 7 has remained OOS since 04 June 2007 for turbine rehab.

1.3.4 Unit 8 was OOS from 0015 on 13 April until 1602 on 15 April due to Bank 7/8 outage.
1.3.5 Unit 9 has remained OOS since 04 August for turbine rehab.
1.3.6 Unit 11 has remained OOS since 12 June due to rotor damage.
1.3.7 Unit 12 returned to service at 1131 on 13 April.  It had been OOS since 9 April due to exciter trouble.
1.3.8 Unit 12 was OOS from 2007 until 2308 on 15 April for T11 wiring/BPA trip testing.

1.3.9 Unit 13 was OOS from 0947 until 1343 on 13 April for gland water/packing.
1.3.10 Unit 13 was OOS from 2008 until 2303 on 15 April for T11 wiring/BPA trip testing.

1.3.11 Unit 14 was OOS from 2008 until 2302 on 15 April for T11 wiring/BPA trip testing.

1.3.12 Unit 15 was OOS from 2001 on 16 April until 0050 on 17 April for T12 wiring/BPA trip testing.

1.3.13 Unit 16 was OOS from 2003 on 16 April until 0046 on 17 April for T12 wiring/BPA trip testing.

1.3.14 Unit 17 was OOS from 2004 on 16 April until 0043 on 17 April for T12 wiring/BPA trip testing.

1.3.15 Unit 18 was OOS from 2008 on 16 April until 0044 on 17 April for T12 wiring/BPA trip testing.

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.
1.4.1 F1 was OOS from 2002 until 2321 on 15 April for T11 wiring/BPA trip testing.
1.4.2 F2 was OOS from 1956 on 16 April until 0042 on 17 April for T12 wiring/BPA trip testing.

1.5 Gatewell drawdowns are measured on Wednesdays.
2 ACTIVITIES

2.1 Auxiliary Water System Closures.  The following AWS valves were closed this reporting week.
2.1.1 FV1-1 was closed for cleaning from 1620 until 1633 on 16 April.

2.1.2 FV3-9 was closed for cleaning from 0832 until 0847 on 13 April.

2.1.3 FV5-9 was closed for cleaning from 1608 until 1614 on 16 April.

2.1.4 FV6-9 was closed for cleaning from 1608 until 1614 on 16 April.

2.2 STS/VBS inspections.  The following STS/VBS activities occurred this reporting week:  
Table 1.  STS and VBS inspections.  
	Unit
	Date
	Run Hours
	Comments

	1-2
	NA
	NA
	Remain OOS until 3 September.

	11-18
	NA
	NA
	No inspections this reporting week.


2.3 Dewatering and Fish Salvage.  No fish salvage or dewatering activities occurred this week.
3 RESEARCH  

3.1 CRITFC- JMF video.  Video equipment installation and testing began this week.  More equipment scheduled for installation the week of 10 May. 
3.2 CRITFC - Annual chinook, coho, sockeye and steelhead sampling in the AFF is expected to begin the week of 20 April.
3.3 FFU- sea lion.  Sea lion observations during the day and at night continue.
3.4 NOAA- spill gate test.  Vibration sensors remain installed on Spill bay 6. 

3.5 NOAA- B2CC PIT.  B2CC PIT tag detector testing occurred on 15-16 April.
3.6 NOAA-FGE.  No test releases occurred during the 13-18 April period in order to avoid impacting 6.5 million Spring Creek National Fish Hatchery tules on the scheduled 13 April release.  Testing will resume on 20 April.
3.7 NOAA Lamprey.  Cascades Island lamprey ramp installation (phase two) and de-winterization of Bradford Island and WA shore lamprey passage structures (LPS) scheduled for week of 20 April.
3.8 PNNL- BGS.  All hydrophones are installed.  Hydro-acoustic tagging is scheduled for 20 April and the first release is scheduled on 22 April.  
3.9 PSMFC- Smolt monitoring.  Daily condition and index monitoring continues.  Fallbacks recorded crossing the separator bars included; 15 steelhead and one unknown salmonid.  

3.10 WDFW Fish Counting.  Visual counting continues at Bradford Island and Washington Shore fishways.
3.11 WDFW/ODFW/CRITFC Sea lion hazing.  Hazing from boats and trapping continues. 
3.12 University of Washington.  Barge versus bypass fish testing for latent mortality began 10 April.
3.13 USDA Wildlife Services. Hazing sea lions from shore continues.  Avian hazing began on 01 April.
4 FISHWAYS 

4.1 Project biologists inspected 12 through 18 April.   NOAA inspected on 16 April.
4.2  Fish Passage Plan Observations.
Table 2.  FPP Out of Criteria items.
	DATE
	LOCATion
	FPP itEMS
	CAUSE
	RESPONSE
	TR#

	BRADFORD ISLAND

	4/13
	FG 3-23
	Incorrect position
	In manual
	Ops adjusted.
	

	4/13, 16, 18
	B-branch entr.
	Differential >2.0’
	PLC issues
	Ops adjusted, FV4-3 in manual
	

	4/16
	BI weir
	Reading <1.0’
	Unknown
	Ops adjusted
	

	4/16
	B-branch weir
	Reading <1.0’
	Unknown
	Ops adjusted
	

	CASCADES ISLAND (CI)

	4/16
	UMT weir
	Reading >1.0’
	Unknown
	Ops corrected.
	

	4/13, 18
	CI entrance
	Differential >2.0’
	PLC issues
	Ops placed FV 5-3 in manual
	

	WASHINGTON SHORE

	4/16
	PH2 entrances
	Differential >2.0’
	Entrance gates in manual
	Ops adjusted
	

	DSM1/ITS – DSM1 is OOS until early September

	DSM2/ JMF


4.2.1 PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Repairs scheduled for 09/10.  

4.2.2 Design of a mechanical velocity meter at the PH2 south monolith is in progress.  

4.3 Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington shore.
Table 3.  Bonneville Fishway Temperatures

	Date
	WA. Shore average
	BI

 average

	4/12
	NA
	45.5

	4/13
	NA
	45.7

	4/14
	NA
	45.5

	4/15
	NA
	45.7

	4/16
	NA
	46.5

	4/17
	NA
	47.4

	4/18
	NA
	48.0


4.4 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.  
4.5 Adult Fishways. 
4.5.1 Calibration was checked on 17 April.  
4.5.2 The Bradford Island exit was closed for cleaning from 0832 until 0847 on 13 April.

4.5.3 B-branch and Cascades Island PLC’s switched between auto and manual throughout the week due to programming issues.  A programmer was on site on 18 April.  B-branch and CI PLC’s currently remain in auto.
4.5.4 The geotechnical drilling at the Cascades Island fishway concluded on 9 April.  Data loggers are installed in the holes to determine if there is water movement.

4.5.5 The Cascades Island south entrance gate remains closed.  It is only out of service when tailwater is below 17’.

4.5.6 The Washington Shore fishway entrances are in manual due to SA-24 board calibration issues.  The operators are adjusting the gates to maintain at least a 13’ opening.

4.5.7 All 12 FOGs are now in service.
4.5.8 The AFF is expected to return to service for sampling the week of 20 April. 
4.6 Juvenile Fishways.
4.6.1 Spillbay 17 remains on two dogs.  The bay should be at 3.5’ but due to a broken hoist it has to be dogged instead.  Flow is compensated through adjacent bays.
4.6.2 DSM2 mechanical screen cleaner went out of service at 1425 on 17 April due to a broken chain.  It was repaired on 20 April and placed back in service.
4.7 Lamprey Fishways.  BI and WS AWS lamprey ramps remain winterized.
4.7.1 The new Cascades Island weir is scheduled for installation in early May.
5 CONSTRUCTION
5.1 PH1 Ice and Trash Sluiceway Wall Removal.  Wall removal is scheduled for FY10.
5.2 TIE crane.  The TIE crane boom repair has been delayed.
6 HAZMAT, SPILLS AND CLEANUP.   Nothing to report.
7 TELETYPES AND OTHER NOTEWORTHY ITEMS
7.1 A teletype sent on 06 November set the BON upper forebay limit as 76.5’ msl and the lower limit as 71.5’ msl.  

7.2 A teletype sent on 9 April requests spring spill to begin at 0001 on 10 April and to continue until further notice.

7.3 A teletype sent on 13 April requests operation of Powerhouse 2 at the lower end of the 1% efficiency band to minimize impact to the 6.5 million juvenile fall Chinook released from the Spring Creek hatchery. The request is a hard constraint from 14 April through 18 April and a soft constraint from 18 April until 22 April.

7.4 A teletype sent on 16 April requests that Bonneville forebay elevation be maintained between 72.0’ and 73.0’ msl as a soft constraint from 1100 to 1500 hrs on 25 April for debris removal from the shoreline in Stevenson, WA.

7.5 A teletype sent on 17 April revises the Spring Creek hatchery special operations teletype sent on 13 April. The soft constraint component of that teletype was eliminated due to monitoring that showed those fish had passed Bonneville project and special powerhouse operations were no longer necessary.
JAMES R. MAHAR, P.E.

Operations Manager
1

