CENWP-OD-B                                                                                                      


25 May 2009 
MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist
SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 21 of 2009, which covers the period from 17 through 23 May 2009.
1 OPERATION SUMMARY

1.1 Daily average river flows ranged from 255.9 to 329.3 kcfs.  Daily average project tailwater ranged from 20.2’ to 24.2’ msl.  Daily average powerhouse forebay elevation ranged from 73.4’ to 74.9’ msl.  Daily average river temperature ranged from 54ºF to 56ºF.  Secchi disk measurements ranged from 3.0’ to 4.0’.
1.2 Daily average spill ranged from 84.9 to 139.0 kcfs.
1.2.1 Bay 17 remains set at two dogs (2.9 feet).  

1.3 Unit Operation.  Powerhouse 2 remains the priority powerhouse.
1.3.1 Unit 5 was out of service from 1536 until 1556 on 21 May for switching.

1.3.2 Unit 6 was out of service from 1536 until 1553 on 21 May for switching.

1.3.3 Unit 7 has remained out of service since 04 June 2007 for turbine rehab.
1.3.4 Unit 8 was out of service from 0032 on 18 May until 1555 on 21 May for bank 7/8 outage.  
1.3.5 Unit 9 has remained out of service since 04 August for turbine rehab.
1.3.6 Unit 11 has remained out of service since 12 June due to rotor damage.
1.3.7 Unit 16 went out of service on 09 May due to an emergency shutdown caused by exciter relay troubles and a grounding issue.

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units are placed on standby, as necessary, during nighttime hours to float debris.
1.5 Gatewell drawdowns are measured on Wednesdays.
2 ACTIVITIES

2.1 Auxiliary Water System Closures.  The following AWS valves were closed this reporting week:
2.1.1 FV 1-1 was closed for cleaning from 0715 until 0725 on 21 May.

2.1.2 FV 3-7 was closed for cleaning from 0700 until 0710 on 21 May.

2.1.3 FV 3-9 was closed for cleaning from 0700 until 0710 on 21 May.

2.1.4 FV 6-9 was closed for cleaning from 0739 until 0745 on 21 May.

2.2 STS/VBS inspections.  The following STS/VBS inspections occurred this reporting week:  
Table 1.  STS and VBS inspections.  
	Unit
	Date
	Run Hours
	Comments

	1-2
	NA
	NA
	Remain out of service until 3 September.

	11
	NA
	NA
	Currently out of service

	14 
	5/19
	NA
	Gatewell 14A visually inspected prior to NMFS gatewell fish releases

	12-13, 15-18
	5/21
	NA
	 VBS visually inspected and cleaned


2.2.1      Debris on VBSs has been increasing with increased river flows.  They are being closely monitored  and a crew was brought in over the holiday weekend to clean VBSs and keep drawdown within FPP criteria.

2.3 Dewatering and Fish Salvage.  No fish salvage or dewatering activities occurred this week.
3 RESEARCH  

3.1 CRITFC - Annual chinook, coho, sockeye and steelhead sampling in the AFF continues.

3.2 CRITFC- JMF video.  Video equipment for monitoring juvenile fish passage on hold until new equipment arrives.  
3.3 FFU- sea lion.  Sea lion observations continue.
3.4 NOAA-FGE.  Test releases of PIT-tagged run of the river fish continued this week to assess passage conditions at two turbine operating ranges (middle and upper end of the 1% efficiency range).  Test fish were released at the intake into gatewell 14A.  2,032 yearling chinook collected from the JMF.
3.5 NOAA Lamprey.  Water-up for all lamprey passage structures scheduled for the week of 25 May.  
3.6 NOAA- pathogens. Determination of pathogen prevalence in juvenile salmonids.  Collected 25 yearling chinook and 25 coho from the JMF.

3.7 NOAA- PIT tag detectors.  Vibration sensors remain installed on Spill bay 6. 

3.8 PNNL- BGS.  All hydrophones are installed and collecting data for the BGS evaluation.  
3.9 PSMFC- Smolt monitoring.  Daily condition and index monitoring continues.  Fallbacks recorded crossing the separator bars included; 81 steelhead, 27 salmon, and four unknown salmonids.  Gas Bubble Trauma (GBT) examinations occur two days per week at the JMF during spill.  Please go to http://www.fpc.org/CurrentDaily/Gbtsum.txt for further details. 2,032 yearling chinook held for NOAA gatewell 14A testing.  84 yearling chinook were collected for University of Washington studies and 25 yearling chinook and 25 coho were held for NOAA this week.

3.10 WDFW Fish Counting.  Visual counting continues at Bradford Island and Washington Shore fishways.
3.11 WDFW/ODFW/CRITFC Sea lion hazing.  Hazing from boats and trapping concluded. 
3.12 USDA Wildlife Services. Hazing sea lions from shore concluded.  Avian hazing continues.

3.13 University of Idaho.  Tagging adult spring Chinook in the AFF for evaluation of the Cascades Is. entrance weir.
3.14 University of Washington.  Barge versus bypass fish testing for latent mortality continues.  Collected 84 yearling chinook from the JMF.
4 FISHWAYS 

4.1 Project biologists inspected 17 through 23 May.  
4.2  Fish Passage Plan Observations.
Table 2.  FPP Out of Criteria items.
	DATE
	LOCATion
	FPP itEMS
	CAUSE
	RESPONSE
	TR#

	BRADFORD ISLAND – nothing out of criteria

	CASCADES ISLAND (CI) – nothing out of criteria

	WASHINGTON SHORE – nothing out of criteria

	ITS – nothing out of criteria

	DSM2/ JMF- nothing out of criteria


4.2.1 PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Repairs scheduled for 09/10.  

4.2.2 Design of a mechanical velocity meter at the PH2 south monolith is in progress.  

4.2.3 CI south entrance gate remains broken and closed.  It is been OOS since 10 April.  It will remain in the closed position for all tailwater elevations during the modified entrance evaluation. 
4.3 Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island and at the junction of the UMT and the main ladder at Washington Shore.  

4.3.1 The WA shore and Bradford Is. probes need calibration; until then the AFF probe will be used as a surrogate.
Table 3.  Bonneville Fishway Temperatures

	Date
	AFF average

	5/17
	53.4

	5/18
	54.2

	5/19
	54.3

	5/20
	54.2

	5/21
	54.5

	5/22
	55.3

	5/23
	56.1


4.4 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.  
4.5 Adult Fishways. 
4.5.1 Calibration was checked on 21 May.  
4.5.2 The Washington Shore fishway entrances are in manual due to SA-24 board calibration issues.  The operators are adjusting the gates to maintain at least a 13’ opening.
4.5.3 There are currently 11 of 12 floating orifice gates (FOG) installed at Powerhouse 2.  The last gate was removed for maintenance and replaced with a bulkhead.
4.6 Juvenile Fishways.
4.6.1 Spillbay 17 remains on two dogs (2.9’).  The bay should be at 3.5’ but due to a broken hoist it has to be dogged instead.  Flow is compensated through adjacent bays.
4.7 Lamprey Fishways.  
4.7.1 The CI entrance weir is installed.
4.7.2 All lamprey passage structures are scheduled for water-up during the week of 25 May.  The NDE lamprey ramp is scheduled for installation on 27 May.  NDE gate will be closed briefly to facilitate the ramp deployment. 
5 CONSTRUCTION
5.1 PH1 Ice and Trash Sluiceway Wall Removal.  Wall removal is scheduled for FY10.  Old DSM bypass equipment in the tailrace is scheduled for removal in FY10 as well.
5.1.1 The electrical work for the automated gates is completed and all that remains is setting up the programmable logic controller (PLC).
5.2 TIE crane.  The new boom is to arrive in mid-June; load testing should occur by the first week in July.
6 HAZMAT, SPILLS AND CLEANUP.   Nothing to report.
7 TELETYPES AND OTHER NOTEWORTHY ITEMS
7.1 A teletype sent on 09 April requests spring spill to begin at 0001 on 10 April, continuing until further notice.

7.2 A teletype sent on 17 May and revised on 18 May provides guidance for voluntary and involuntary spill by project as well as spill caps per project for various total dissolved gas (TDG) levels.

7.3 A teletype sent on 18 May requests a forebay elevation soft constraint of 72.0-73.5’  from 1000 to 1200 hrs on 23 May to facilitate the cleanup of the beach at the Port of Hood River.
JAMES R. MAHAR, P.E.

Operations Manager
Page 1 of 3
1
Page 2 of 3

