CENWP-OD-B






















4 January 2010
MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist
SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 01 of 2010, which covers the period from 26 December 2009 through 02 January 2010.
1 OPERATION SUMMARY

1.1 Daily average river flows ranged from 105.9 to 141.0 kcfs.  Daily average project tailwater ranged from 12.0’ to 12.7’ msl.  Daily average powerhouse forebay elevation ranged from 75.5’ to 77.1’ msl.  Daily average river temperature ranged between 38ºF and 39ºF.  Secchi disk measurements ranged from 6.0’ to 7.0’.
1.2 Spill Bays 1, which provides attraction flow for Cascades Island, is open 6” during daylight hours.  Daily average spill ranged from 0.5 to 0.6 kcfs.
1.3 Unit Operation.  Powerhouse Two is the priority powerhouse.
1.3.1 Unit 1 returned to service at 1406 on 28 December.  It had been of service since 21 December for DSM1 orifice removal.

1.3.2 Unit 7 has remained out of service since 04 June 2007 for turbine rehab.
1.3.3 Unit 9 has remained out of service since 04 August 2008 for turbine rehab.
1.3.4 Unit 11 has remained out of service since 12 June 2008 due to rotor damage.
1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.
1.4.1 F1 remains out of service for annual maintenance.
1.5 Gatewell drawdowns are measured on Wednesdays.
2 ACTIVITIES

2.1 Auxiliary Water System Closures.  The following AWS valves were closed this reporting week: 

2.1.1 FV1-1, 3-7, 3-9, 4-3, and 4-4 remain closed for winter maintenance in Bradford Island.
2.1.2 FV5-9- closed for cleaning from 1120 until 1135 on 28 December.
2.1.3 FV6-9- closed for cleaning from 1120 until 1135 on 28 December.
2.2 STS/VBS inspections.  The following STS/VBS inspections and removal occurred this reporting week.
Table 1.  STS and VBS inspections.  
	Unit
	Date
	Run Hours
	Comments

	11
	NA
	OOS
	STSs removed on 28 August 2008.

	12-18
	NA
	
	STSs scheduled for installation the week of 21 February.


2.3 Dewatering and Fish Salvage.  No dewatering/fish salvage activities occurred this week.
3 RESEARCH  

3.1 NOAA- PIT tag detectors.  Vibration sensor remains installed on Spill bay 6. 

3.2 ODFW/WDFW sea lions.  Moved the forebay sea lion trap on 19 November.
3.3 Smith-Root sea lion array.  Array remains installed in the UMT.
3.4 WDFW Fish Counting.  Video counting at Washington Shore continues.  
4 FISHWAYS 

4.1 Project biologists inspected 28, 29, 31 December and 02 January.
4.2  Fish Passage Plan Observations.
Table 2.  FPP Out of Criteria items.
	DATE
	LOCATion
	FPP itEMS
	CAUSE
	RESPONSE
	TR#

	BRADFORD ISLAND (BI)- fishway currently dewatered

	CASCADES ISLAND (CI)- nothing to report

	
	
	
	
	
	

	WASHINGTON SHORE (WS) – nothing to report


4.2.1 PH1 PCC diffusers FG2-22A and FG2-19 are closed or partially open.  Repairs scheduled for 2009/2010 in water work period.  

4.2.2 Design of a mechanical velocity meter at the PH2 south monolith is in progress.  

4.3 Fishway Temperatures.  Measurements are taken three weirs down from the count station at Bradford Island; the junction of the UMT and the Washington Shore ladder.  

4.3.1 Temperatures are taken 01 March through 30 November. 
Table 3.  Bonneville Fishway Temperatures
	Date
	WS Avg
	BI Avg

	12/1-2/28
	NA
	NA


4.4 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.  
4.5 Adult Fishways. 
4.5.1 Calibration was checked on 31 December.
4.5.2 The PH1 ice and trash sluiceway (ITS) remains dewatered.
4.5.2.1 The wooden bulkhead is scheduled for replacement this winter.

4.5.3 The Washington Shore fishway entrances remain in manual due to SA-24 board calibration issues.  
4.5.3.1 NUE remains closed.  The other gates are adjusted to maintain a 13’ opening, as tailwater allows.
4.5.4 Stab plates remain installed in 11 of 12 FOGs.  One gate remains out for maintenance, with a bulkhead installed.
4.5.5 SLEDs were installed at PH2 entrances by 1400 on 29 December.

4.6 Juvenile Fishways.
4.6.1 The B2CC remains closed.
4.6.2 DSM2 remains dewatered.
4.7 Lamprey Fishways.  All LPSs have been dewatered.
4.7.1 The Cascades Is. entrance weir and slot filler remain installed.  
4.7.2 The PH2 NDE ramp remains in the lowered position.
5 CONSTRUCTION
5.1 PH1 Gantry 4.  Crane rails replacement continues.  New crane in FY11.  
5.2 PH1 Ice and Trash Sluiceway Wall Removal.  Wall removal continues.  Outfall removal has been cancelled.
5.3 TIE crane.  Load testing occurred the week of 29 November.
6 HAZMAT, SPILLS AND CLEANUP.   Nothing to report.
7 TELETYPES AND OTHER NOTEWORTHY ITEMS
7.1 A teletype sent on 16 July 2009, changes PH2 unit priority to: 12,18,15,17,14,13,16.  This change is effective from 1100 on 16 July 2009 until U11 returns to service.
7.2 A teletype sent on 01 September requests BON forebay maintain 71.5’ to 78.0’ msl as a hard constraint and 71.5’ to 77.5’ as a soft constraint from 0001 on 08 September 2009 until 31 March 2010 to support chum spawning below the dam.  

7.3 A teletype sent on 08 September requests that BON operate the single highest priority unit at PH1 (currently Unit 1) as needed.  This is to reduce the risk of BON going outside the 1% efficiency range while supporting the high forebay request for TDA spillwall work.  This request is effective immediately until further notice.
7.4 A teletype sent on 05 November requests BON tailwater elevation hard constraint of no less than 11.3’ msl during all hours and a soft constraint of 11.3’ to 11.7’, with a target of 11.5’, from 0600 until 1700 daily.  This is effective from 0600 on 06 November until further notice.  This request is to provide necessary flows for chum spawning.  
7.4.1 A teletype sent on 30 December modified the 05 November teletype.  The new teletype requests BON tailwater elevation be maintained at no less than 11.5’ msl during all hours.  This is effective from 1600 on 30 December until further notice.  This request is to provide necessary flows from chum egg incubation.

JERRY CARROLL
Acting Operations Manager
Page 1 of 2
1
Page 2 of 2

