CENWP-OD-B






















18 May 2010
MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist
SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 20 of 2010, which covers the period from 09 through 15 May 2010.
1 OPERATION SUMMARY

1.1 Daily average river flows ranged from 159.8 to 209.1 kcfs.  Daily average project tailwater ranged from 15.3’ to 17.4’ msl.  Daily average powerhouse forebay elevation ranged from 75.0’ to 75.5’ msl.  Daily average river temperature ranged from 52 to 53ºF.  Secchi disk measurements ranged from 4.0’ to 5.0’.
1.2 Spill Bays 1 and 18, which provide attraction flow for Cascades Island and B-branch, respectively, are open 6” during daylight hours.  Daily average spill ranged from 74.4 to 89.5 kcfs.
1.3 Unit Operation.  Powerhouse Two is the priority powerhouse. 
1.3.1 Unit 7 has remained out of service since 04 June 2007 for turbine rehab.

1.3.2 Unit 9 has remained out of service since 04 August 2008 for turbine rehab.
1.3.3 Unit 11 has remained out of service since 12 June 2008 due to rotor damage. 

1.3.4 Unit 14 was out of service from 0745 until 1038 on 13 May due to governor speed signal failure.

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.
1.5 Gatewell drawdowns are measured on Wednesdays.
2 ACTIVITIES

2.1 Auxiliary Water System Closures.  No AWS valves were closed this reporting week:
2.2 STS/VBS inspections.  No STS/VBS inspections occurred this reporting week:
Table 1.  STS and VBS inspections.  
	Unit
	Date
	Run Hours
	Comments

	11
	NA
	OOS
	U11 remains out of service.

	12-18
	NA
	NA
	


2.3 Dewatering and Fish Salvage.  The following dewatering/fish salvage activities occurred this week: 

2.3.1 Unit 7 tail logs were pulled on 11 May.  One decayed adult chinook was returned to the river.
3 RESEARCH  

3.1 Battelle – survival study.  Hydrophone installation in PH1, PH2, and spillway forebays complete. Tagged fish releases upstream have begun for dam passage and survival evaluations. Sort by Code operations at the SMF continued this week to evaluate acoustic tagging effects on juvenile salmonids.
3.2 CRITFC- sea lion. Deployed hydrophones on 01 March in BON tailraces to track sea lion activity.

3.3 CRITFC - Annual chinook, coho, sockeye and steelhead sampling in the AFF continues.

3.4 FFU- sea lion.  Observations continue.  Weekly reports of sea lion activities are now available. If you would like to be included on the distribution list contact Robert Stansell-541-374-8801. 

3.5 NOAA- lamprey.  Cascades Island lamprey ramp camera remains installed.
3.6 PSMFC- smolt monitoring.  Juvenile fish condition and index monitoring continues.  Fallbacks recorded crossing the separator bars for this reporting week:  151 steelhead, 28 salmon, and 12 unknown salmonids.  Gas Bubble Trauma (GBT) examinations occur two days per week at the JMF during spill.  Please go to http://www.fpc.org/CurrentDaily/Gbtsum.txt for further details.  Two juvenile lamprey collected for University of Idaho researchers this week.  
3.7 WDFW- Fish Counting.  Visual counting continues.
3.8  WDFW, ODFW, CRITFC- Sea lion hazing/trapping.  Hazing from boats continues.  Sea lion traps remain in position.  Trapping continues.
3.9  USDA Wildlife Services- Sea lion hazing.  Hazing from shore continues.

3.10 USDA Wildlife Services- Avian hazing.  Hazing from shore continues.

3.11 University of Idaho – lamprey.   Two juvenile lamprey collected from PSMFC SMP this reporting week. 

3.12 University of Idaho - Adult salmon sampling continues for monitoring passage effects at the modified Cascades Island entrance.
4 FISHWAYS 

4.1 Project biologists inspected 09-15 May.
4.2  Fish Passage Plan Observations.
Table 2.  FPP Out of Criteria items.
	DATE
	LOCATion
	FPP itEMS
	CAUSE
	RESPONSE
	TR#

	Once
	BON
	Calibration not done
	Personnel shortage
	To be completed week of 5/16
	

	BRADFORD ISLAND (BI) – Nothing to report

	CASCADES ISLAND (CI) – Nothing to report

	WASHINGTON SHORE (WS)

	5/9-11, 15
	PH2 entrances
	Gates <13.0 ft open
	System in manual
	Operators adjusted
	


4.2.1 PH1 PCC diffuser FG2-19 is partially open.  Repairs scheduled for 2011/2012 in water work period.  

4.2.2 Design of a mechanical velocity meter at the PH2 south monolith is in progress.  

4.2.3 The SA-24 board that controls the Powerhouse 2 adult ladder components is in need of replacement. A new programmable logic controller (PLC) system has been budgeted for.
4.3 Fishway Temperatures.  Temperatures are taken 01 March through 30 November.

Table 3.  Bonneville Fishway Temperatures

	Date
	WA. Shore average
	BI

average

	5/9
	51.5
	52.5

	5/10
	51.7
	52.6

	5/11
	51.7
	52.6

	5/12
	51.7
	52.6

	5/13
	52.1
	53.1

	5/14
	53.0
	54.0

	5/15
	53.8
	54.7


4.4 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.  
4.5 Adult Fishways. 
4.5.1 Calibration was not checked this week.
4.5.2 The Washington Shore fishway entrances remain in manual due to SA-24 board calibration issues.  The operators are adjusting the gates to maintain at least a 13’ opening, as tailwater allows.
4.5.3 Sea lion exclusion bars remain in 11 of 12 floating orifice gates (FOGs).  One FOG remains out for maintenance, with a bulkhead installed in its place.
4.5.4 Sea Lion Exclusion Devices (SLEDs) remain installed at all fishway entrances. 
4.5.5 Sampling at the AFF continues.
4.6 Juvenile Fishways.
4.6.1 DSM2 remains in service.
4.6.2 The PH1 ice and trash sluiceway (ITS) remains in service.  
4.6.3 Hydrocannon remains in service.
4.6.4 TIES are installed.
4.7 Lamprey Fishways.  All LPSs are dewatered.
4.7.1 The Cascades Is. entrance weir remains installed.  
4.7.2 The PH2 NDE ramp remains in the lowered position.
5 CONSTRUCTION
5.1 PH1 Gantry 4.  New crane in FY11.  
5.2 Spillway Gate Repair Pit. Clean-up of hazardous material is in progress.  Shoring of the walls is scheduled to occur in late summer.
6 HAZMAT, SPILLS AND CLEANUP.  Nothing to report.
7 TELETYPES AND OTHER NOTEWORTHY ITEMS
7.1 A teletype sent on 16 July 2009, changes PH2 unit priority to: 12,18,15,17,14,13,16.  This change is effective from 1100 on 16 July 2009 until U11 returns to service.
7.2 A teletype sent on 09 April requests spill for juvenile fish passage begin at 0001 hours on Saturday, 10 April and continue until further notice. Spill approximately 100 kcfs 24 hours a day using the spill pattern in the 2010 FPP.  If Bonneville inflows are too low to maintain the 100 kcfs spill and minimum turbine discharge of 30 kcfs, then operate to minimum discharge and spill to pass inflow.  

7.3 A teletype sent on 07 May requests special operations for the 4.65 million Spring Creek hatchery Chinook releases on 10 May.  Beginning 0600 on 11 May until 0559 on 15 May as a hard constraint, and 0600 on 15 May until 0559 on 19 May as a soft constraint - Operate PH2 as priority with units operating within 5 MW of the low 1% limit as a hard constraint.  PH1 units shall operate within the 1% best efficiency range.  
7.4 A teletype sent on 10 May requests that Bonneville forebay be operated for the treaty fishery as follows:  0600 Tuesday, 11 May until 1800 Friday, 14 May – operate within 1.5’ band, hard constraint.  This is to limit pool fluctuation and reduce wear and tear on equipment.  

GLEN A. SMITH, P.E.
Operations Project Manager
Page 1 of 3
1
Page 2 of 3

