CENWP-OD-B






















02 August 2010
MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist
SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 31 of 2010, which covers the period from 25 through 31 July 2010.
1 OPERATION SUMMARY

1.1 Daily average river flows ranged from 130.2 to 165.3 kcfs.  Daily average project tailwater ranged from 12.0’ to 14.4’ msl.  Daily average powerhouse forebay elevation ranged from 74.9’ to 75.7’ msl.  Daily average river temperature ranged from 70 to 71ºF.  Secchi disk measurements ranged from 4.5’ to 6.8’.
1.2 Spill Bays 1 and 18, which provide attraction flow for Cascades Island and B-branch, respectively, are open 6” during daylight hours.  Daily average spill ranged from 79.9 to 88.6 kcfs.
1.3 Unit Operation.  Powerhouse Two is the priority powerhouse. 
1.3.1 Unit 9 has remained out of service since 04 August 2008 for turbine rehab.
1.3.2 Unit 11 has remained out of service since 12 June 2008 due to rotor damage.
1.3.3 Unit 13 went out of service at 0733 on 14 June for four-year overhaul.
1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.
1.5 Gatewell drawdowns are measured on Wednesdays.
2 ACTIVITIES

2.1 Auxiliary Water System Closures.  No AWS valves were closed this reporting week.
2.2 STS/VBS inspections.  The following STS/VBS inspections occurred this reporting week:
Table 1.  STS and VBS inspections.  
	Unit
	Date
	Run Hours
	Comments

	11
	NA
	OOS
	U11 remains out of service.

	12
	7/26
	570
	OK

	13
	7/26
	OOS
	Out of service.

	14
	7/26
	469
	OK

	15
	7/26
	561
	OK

	16
	7/26
	145
	OK

	17
	7/26
	481
	OK

	18
	7/26
	549
	OK


2.3 Dewatering and Fish Salvage.  No dewatering/fish salvage activities occurred this week.
3 RESEARCH  

3.1 Battelle – survival study.  Hydrophone installation in PH1, PH2, and spillway forebays complete. Tagged fish releases upstream continue for dam passage and survival evaluations. Sort by Code operations at the SMF continued this week to evaluate acoustic tagging effects on juvenile salmonids. 
3.2 CRITFC - Annual chinook, coho, sockeye and steelhead sampling in the AFF continues.

3.3 NOAA- lamprey.  PIT tagging adult lamprey in the AFF continues.  
3.4 PSMFC- smolt monitoring.  Juvenile fish condition and index monitoring continues.  Fallbacks recorded crossing the separator bars for this reporting week:  one steelhead.  Gas Bubble Trauma (GBT) examinations occur two days per week at the JMF during spill.  Please go to http://www.fpc.org/CurrentDaily/Gbtsum.txt for further details.  
3.5 WDFW- Fish Counting.  Visual counting continues.  Night counts continue.
3.6 USDA Wildlife Services- Avian hazing.  Hazing from shore continues.

3.7 University of Idaho – lamprey.   Radio and PIT tagging adult lamprey in the AFF.
4 FISHWAYS 

4.1 Project biologists inspected 25-31 July. 
4.2  Fish Passage Plan Observations.
Table 2.  FPP Out of Criteria items.
	DATE
	LOCATion
	FPP itEMS
	CAUSE
	RESPONSE
	TR#

	BRADFORD ISLAND (BI)

	7/25
	B-branch weir
	Reading < 1.0’
	Unknown
	Operators adjusted
	

	7/28
	A-branch weir
	Reading < 1.0’
	Unknown
	Operators adjusted
	

	7/28
	BI weir
	Reading < 1.0’
	Unknown
	Operators adjusted
	

	All week
	Chain gate 6C
	Incorrect position
	Slack cable trouble
	See 4.6.5
	10-184724

	CASCADES ISLAND (CI) Nothing to report.

	WASHINGTON SHORE (WS)

	7/25,31
	PH2 entrance gates
	Submerged <13’
	In manual
	Operators adjusted
	


4.2.1 PH1 PCC diffuser FG2-19 is partially open.  Repairs scheduled for 2011/2012 in water work period.  

4.2.2 The SA-24 board that controls the Powerhouse 2 adult ladder components is in need of replacement. A new programmable logic controller (PLC) system has been budgeted for.
4.3 Fishway Temperatures.  Temperatures are taken 01 March through 30 November.
Table 3.  Bonneville Fishway Temperatures

	Date
	WA. Shore average
	BI

average
	AFF average
	SMF average

	7/25
	68.8
	68.9
	68.6
	68.8

	7/26
	69.0
	69.1
	68.6
	69.1

	7/27
	69.3
	69.4
	69.1
	69.4

	7/28
	69.4
	69.4
	69.2
	69.5

	7/29
	69.1
	69.2
	68.9
	69.2

	7/30
	68.8
	69.0
	68.7
	69.0

	7/31
	68.8
	68.9
	68.6
	69.0


4.4 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.  
4.5 Adult Fishways. 
4.5.1 Calibration was checked on 30 July.
4.5.2 The Washington Shore fishway entrances remain in manual due to SA-24 board calibration issues.  The operators are adjusting the gates to maintain at least a 13’ opening, as tailwater allows.
4.5.3 Sea lion exclusion bars remain in 11 of 12 floating orifice gates (FOGs).  One FOG remains out for maintenance, with a bulkhead installed in its place.

4.5.4 Sampling at the AFF continues.
4.5.5 Operators continue to maintain 0.5’ of head at the PH2 fishway entrances for lamprey passage during night hours.  Table Bon-5 of the FPP is being utilized as a guideline. 
4.6 Juvenile Fishways.
4.6.1 DSM2 remains in service.
4.6.2 The PH1 ice and trash sluiceway (ITS) remains in service.  PH1 ITS Chain gate 6C continues to stick at elevation 72.8’ msl due to slack cable troubles. As forebay elevations fluctuate, the gate does not adjust as programmed.
4.6.3 Hydrocannon remains in service.
4.6.4 TIES remain installed.
4.7 Lamprey Fishways.
4.7.1 The Cascades Is. entrance weir and slot filler remain installed.  Cascade Is. LPS remains in service. Scheduled to operate Sunday night through Friday.
4.7.2 The PH2 NDE ramp remains in the lowered position and in service. 
4.7.3 Bradford Island and WA Shore LPS’s remain in service.
5 CONSTRUCTION
5.1 PH1 Gantry 4.  New crane in FY11.  
5.2 Spillway Gate Repair Pit. Clean-up of hazardous material is in progress.  Shoring of the walls is scheduled to begin in 09 August.
6 HAZMAT, SPILLS AND CLEANUP.  Nothing to report.
7 TELETYPES AND OTHER NOTEWORTHY ITEMS
7.1 A teletype sent on 16 July 2009, changes PH2 unit priority to: 12,18,15,17,14,13,16.  This change is effective from 1100 on 16 July 2009 until U11 returns to service.
7.2 A teletype sent on 16 July requests BON project begin the summer spill at 0430 on 21 July until 2400 on 31 August.  Spill 75kcfs during the day and gas cap at night.  Day time hours are defined in table BON-5 of the 2010 FPP.  Spill in accordance with pattern in table BON-16 of the 2010 FPP.  If flows are insufficient, the project is requested to reduce to minimum generation at night and spill the remainder.  The minimum generation requirement is 30 kcfs, and the minimum spill level is 50 kcfs
7.3 A teletype sent on 23 July requests that Bonneville forebay be operated for the treaty fishery as follows:  0600 Tuesday, 27 July until 1800 Thursday, 29 July – operate within 1.5’ band, hard constraint.  This is to limit pool fluctuation and reduce wear and tear on equipment.
7.4 A teletype sent on 29 July requests BON project change PH1 unit priority as follows: 1, 3,7,6,2,4,5,8,10,9, effective immediately until further notice.  This moves Unit 7 to third in priority.  This operation is necessary to complete a 100 day operation per PH1 rehab contract.  The project is requested to close automatic chain gate 6C and open manual gate 7A to minimize impacts to fish passage.
GLEN A. SMITH, P.E.
Operations Project Manager
Page 1 of 3
1
Page 2 of 3

