CENWP-OD-B





















11 October 2010
MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist
SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 41 of 2010, which covers the period from 03 through 09 October 2010.
1 OPERATION SUMMARY

1.1 Daily average river flows ranged from 80.7 to 120.5 kcfs.  Daily average project tailwater ranged from 7.8’ to 11.6’ msl.  Daily average powerhouse forebay elevation ranged from 74.7’ to 75.7’ msl.  Daily average river temperature ranged from 64 to 67ºF.  Secchi disk measurements ranged from 6.0 to 7.0’.
1.2 Spill Bays 1 and 18, which provide attraction flow for Cascades Island and B-branch, respectively, are open 6” during daylight hours.  Daily average spill ranged from 1.2 to 1.4 kcfs.
1.3 Unit Operation.  Powerhouse Two is the priority powerhouse. 
1.3.1 Unit 0 has remained out of service since 02 August for five year overhaul.
1.3.2 Unit 1 has remained out of service since 17 September due to sticking wicket gates.  
1.3.3 Unit 2 went out of service at 0920 on 05 October for five year overhaul.

1.3.4 Unit 9 has remained out of service since 04 August 2008 for turbine rehab. 
1.3.5 Unit 11 has remained out of service since 12 June 2008 due to rotor damage.
1.3.6 Unit 12 has remained out of service since 13 September for four year overhaul.

1.3.7 Unit 14 was out of service from 2030 on 26 September until 1556 on 27 September due to unit brake control circuit.  
1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.
1.5 Gatewell drawdowns are measured on Wednesdays.
2 ACTIVITIES

2.1 Auxiliary Water System Closures.  The following AWS valves were closed this reporting week:

2.1.1 FV 1-1 was closed for cleaning from 1527 until 1540 on 07 October.
2.1.2 FV 3-9 was closed for cleaning from 1500 until 1513 on 07 October.
2.2 STS/VBS inspections.  No STS/VBS inspections occurred this reporting week:
Table 1.  STS and VBS inspections.  
	Unit
	Date
	Run Hours
	Comments

	11
	NA
	OOS
	U11 remains out of service.

	12-18
	NA
	NA
	Inspection scheduled for the week of 18 October.


2.3 Dewatering and Fish Salvage. The following fish salvage activities occurred this week:
2.3.1 On 06 October, Fisheries recovered two sturgeon from the Unit 2 draft tube.  Both were released in good condition.
3 RESEARCH  

3.1 CRITFC - Annual chinook, coho, sockeye and steelhead sampling in the AFF continues.  
3.2 PSMFC- smolt monitoring.  Juvenile fish condition and index monitoring continues.  Fallbacks recorded crossing the separator bars for this reporting week: 31 salmon and 3 steelhead.  Truck releases from McNary Dam concluded.  Truck releases from Lower Granite, Little Goose, and Lower Monumental dams continue.
3.3 WDFW- Fish Counting.  Visual counting continues. 
4 FISHWAYS 

4.1 Project biologists inspected 03 through 09 October.
4.2  Fish Passage Plan Observations.
Table 2.  FPP Out of Criteria items.
	DATE
	LOCATion
	FPP itEMS
	CAUSE
	RESPONSE
	TR#

	BRADFORD ISLAND (BI)

	10/3,7
	BI fish exit
	Needs cleaning
	Aquatic vegetation
	Riggers cleaned 10/7
	

	10/5
	A-branch weir
	Reading >1.0’
	Unknown
	No action taken.
	

	          CASCADES ISLAND (CI) – Nothing to report.

	WASHINGTON SHORE (WS) - Nothing to report.


4.2.1 PH1 PCC diffuser FG2-19 is partially open.  Repairs scheduled for 2011/2012 in water work period.  

4.2.2 The SA-24 board that controls the Powerhouse 2 adult ladder components is in need of replacement. A new programmable logic controller (PLC) system has been budgeted for.
4.3 Fishway Temperatures.  Temperatures are taken 01 March through 30 November.

Table 3.  Bonneville Fishway Temperatures

	Date
	WA. Shore average
	BI

average

	10/3
	65.6
	66.1

	10/4
	64.6
	65.1

	10/5
	63.9
	64.3

	10/6
	63.7
	64.0

	10/7
	63.5
	63.8

	10/8
	63.3
	63.7

	10/9
	63.3
	63.7


4.4 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.  
4.5 Adult Fishways. 
4.5.1 Calibration was checked on 09 October.
4.5.2 The Washington Shore fishway entrances remain in manual due to SA-24 board calibration issues.  The operators are adjusting the gates to maintain at least a 13’ opening, as tailwater allows.
4.5.3 Sea lion exclusion bars remain in 11 of 12 floating orifice gates (FOGs).  One FOG remains out for maintenance, with a bulkhead installed in its place.

4.5.4 Sampling at the AFF continues.  
4.5.5 The Bradford Island fish exit was closed for cleaning from 1500 until 1513 on 07 October.
4.6 Juvenile Fishways.
4.6.1 DSM2 remains in service.
4.6.2 B2CC remains closed for the season.
4.6.3 The PH1 ice and trash sluiceway (ITS) remains in service.  Chain gate 6C was closed and 7A opened on 04 August as outlined in the teletype sent on 29 July (See 7.2).  Chain gate 7A is set to 72’ msl.
4.6.4 Hydrocannon was shut off and winterized for the season on 06 October.
4.6.5 Three TIES have been removed.  The remaining TIES are scheduled for removal the week of 10 October.
4.7 Lamprey Fishways.
4.7.1 The Cascades Is. entrance weir and slot filler remain installed.  Cascade Is. LPS remains out of service for the season.
4.7.2 The PH2 NDE ramp remains in the lowered position and is not currently operating.
4.7.3 Bradford Island and WA Shore LPS’s remain in service.
5 CONSTRUCTION
5.1 PH1 Gantry 4.  New crane in FY11.  
5.2 Spillway Gate Repair Pit. Clean-up of hazardous material is complete.  Shoring of the walls within 50’ of the Cascades Island fishway is scheduled to resume when fish counts drop below 1,000.
5.3 PH1 juvenile outfall removal.  Removal work above water line is scheduled to occur on 11 October.  In-water work is scheduled to occur between 1-10 November.
5.4 BGS removal. Scheduled to begin mid-December.
5.5 NDE lamprey modifications. Contractor will begin mobilizing on 15 November.
6 HAZMAT, SPILLS AND CLEANUP. Nothing to report.
7 TELETYPES AND OTHER NOTEWORTHY ITEMS
7.1 A teletype sent on 16 July 2009, changes PH2 unit priority to: 12,18,15,17,14,13,16.  This change is effective from 1100 on 16 July 2009 until U11 returns to service.
7.2 A teletype sent on 29 July requests BON project change PH1 unit priority as follows: 1, 3,7,6,2,4,5,8,10,9, effective immediately until further notice.  This moves Unit 7 to third in priority.  This operation is necessary to complete a 100 day operation per PH1 rehab contract.  The project is requested to close automatic chain gate 6C and open manual gate 7A to minimize impacts to fish passage.
7.3 A teletype sent on 07 October requests BON project limit outflow to 90 kcfs or less from 0700 until 1600 on 11 October.  As a hard constraint, BON outflow should not exceed 100 kcfs during this time.  This operation is requested to support removal of the juvenile bypass release pipe downstream of PH1.
7.4  A teletype sent on 08 October requests Bonneville forebay be operated within 1.5’ band, hard constraint, for treaty fishing as follows:  0600 Monday, 11 October until 1800 Wednesday, 13 October.   This is to limit pool fluctuation and reduce wear and tear on equipment.
7.5 Bonneville Fisheries sent out the following Memorandum’s for the Record:
7.5.1 101007: One chinook mortality found on the Valve 15 trashrack.  The fish had not been sampled. 
GLEN A. SMITH, P.E.
Operations Project Manager
Page 1 of 3
1
Page 2 of 3

