CENWP-OD-B





















31 October 2010
MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist
SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 44 of 2010, which covers the period from 24 through 30 October 2010.
1 OPERATION SUMMARY

1.1 Daily average river flows ranged from 80.9 to 107.8 kcfs.  Daily average project tailwater ranged from 8.7’ to 10.8’ msl.  Daily average powerhouse forebay elevation ranged from 74.2’ to 75.8’ msl.  Daily average river temperature ranged from 58 to 61ºF.  Secchi disk measurements ranged from 4.0’ to 7.0’.
1.2 Spill Bays 1 and 18, which provide attraction flow for Cascades Island and B-branch, respectively, are open 6” during daylight hours.  Daily average spill ranged from 1.0 to 1.3 kcfs.
1.3 Unit Operation.  Powerhouse Two is the priority powerhouse. 
1.3.1 Unit 1 has remained out of service since 17 September due to sticking wicket gates.  
1.3.2 Unit 2 has remained out of service since 05 October for five year overhaul.

1.3.3 Unit 10 was out of service from 1240 until 1410 on 25 October for over-speed sensor linkage repair.
1.3.4 Unit 11 has remained out of service since 12 June 2008 due to rotor damage.
1.3.5 Unit 12 has remained out of service since 13 September for four year overhaul.

1.3.6 Unit 16 was out of service from 1500 until 1621 on 26 October to inspect control oil system.

1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.
1.5 Gatewell drawdowns are measured on Wednesdays.
2 ACTIVITIES

2.1 Auxiliary Water System Closures.  The following AWS valves were closed this reporting week:

2.1.1 FV 3-9 was closed for cleaning from 0726 until 0748 on 25 October.

2.2 STS/VBS inspections.  No STS/VBS inspections occurred this reporting week.
Table 1.  STS and VBS inspections.  
	Unit
	Date
	Run Hours
	Comments

	11
	NA
	OOS
	U11 remains out of service.

	12-18
	NA
	OOS
	Inspection scheduled for the week of 15 November.


2.3 Dewatering and Fish Salvage. The following fish salvage activities occurred this reporting week:
2.3.1 On 27 October, Project biologists recovered 25 sturgeon and four steelhead from the upper section of the AFF and the AFF holding pool.  All were released in good condition.
3 RESEARCH  

3.1 PSMFC- smolt monitoring.  Juvenile fish condition and index monitoring concluded on 29 October.  Fallbacks recorded crossing the separator bars for this reporting week: 29 salmon, nine unknown salmonids, and two steelhead.  Truck releases from Lower Granite, Little Goose, and Lower Monumental dams continue.
3.2 WDFW- Fish Counting.  Visual counting continues. Visual counting is scheduled to conclude on 31 October.  Video counting is scheduled to begin on 01 November.
4 FISHWAYS 

4.1 Project biologists inspected 24 through 30 October.
4.2  Fish Passage Plan Observations.
Table 2.  FPP Out of Criteria items.
	DATE
	LOCATion
	FPP itEMS
	CAUSE
	RESPONSE
	TR#

	BRADFORD ISLAND (BI)

	10/26-28
	FG 3-27
	Incorrect position
	Operating issues
	Maintenance repaired
	11-24525

	10/27-30
	PH1 tailrace
	13 avian wires removed
	PH1 outfall removal work
	Schedule to be reinstalled on 18 Nov.
	

	          CASCADES ISLAND (CI) – Nothing to report.

	WASHINGTON SHORE (WS)

	10/28-30
	Weir 38
	Reading <1.0’
	Unknown
	No action taken.
	

	10/30
	17 B orifice
	Closed
	Valve sticking
	Operators opened
	11-30052


4.2.1 PH1 PCC diffuser FG2-19 is partially open.  Repairs scheduled for 2011/2012 in water work period.  

4.2.2 The SA-24 board that controls the Powerhouse 2 adult ladder components is in need of replacement. A new programmable logic controller (PLC) system has been budgeted for.
4.3 Fishway Temperatures.  Temperatures are taken 01 March through 30 November.

Table 3.  Bonneville Fishway Temperatures

	Date
	WA. Shore average
	BI

average

	10/24
	59.5
	59.9

	10/25
	58.7
	58.9

	10/26
	57.8
	58.1

	10/27 
	57.2
	57.3

	10/28
	57.1
	57.4

	10/29
	57.4
	57.8

	10/30
	57.1
	57.4


4.4 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.  
4.5 Adult Fishways. 
4.5.1 Calibration was checked on 30 October.
4.5.2 The Washington Shore fishway entrances remain in manual due to SA-24 board calibration issues.  The operators are adjusting the gates to maintain at least a 13’ opening, as tailwater allows.
4.5.3 Sea lion exclusion bars remain in 11 of 12 floating orifice gates (FOGs).  One FOG remains out for maintenance, with a bulkhead installed in its place.

4.5.4 The upper section of the AFF was dewatered on 26 October.  The lower section of the AFF is scheduled to be dewatered the week of 01 November.
4.5.5 The Bradford Island fish exit was closed for cleaning from 0726 until 0748 on 25 October, and from 1128 until 1133 on 27 October.
4.6 Juvenile Fishways.
4.6.1 DSM2 remains in service.
4.6.2 B2CC remains closed for the season.
4.6.3 The PH1 ice and trash sluiceway (ITS) remains in service.  Chain gate 6C was closed and 7A opened on 04 August.  Chain gate 7A is set to 72’ msl.
4.6.4 The Hydrocannon remains in service.  It is scheduled to be shut off the week of 01 November.
4.7 Lamprey Fishways.
4.7.1 The Cascades Is. entrance weir and slot filler remain installed.  Cascade Is. LPS remains out of service for the season.
4.7.2 The PH2 NDE ramp remains in the lowered position and is not currently operating.
4.7.3 Bradford Island and WA Shore LPS’s were shut down for the season on 26 October.
5 CONSTRUCTION
5.1 PH1 Gantry 4.  New crane in Spring, 2011.  
5.2 Spillway Gate Repair Pit. Clean-up of hazardous material is complete.  Shoring of the walls within 50’ of the Cascades Island fishway is scheduled to resume when fish counts drop below 1,000.
5.3 PH1 juvenile outfall removal.  Removal work above water line began on 11 October.  In-water work is scheduled to occur between 1-10 November. 
5.4 BGS removal. Scheduled to occur during 2011/2012 winter maintenance.
5.5 NDE lamprey modifications. Contract has been canceled for this in-water work period.

6 HAZMAT, SPILLS AND CLEANUP. Nothing to report.
7 TELETYPES AND OTHER NOTEWORTHY ITEMS
7.1 A teletype sent on 16 July 2009, changes PH2 unit priority to: 12,18,15,17,14,13,16.  This change is effective from 1100 on 16 July 2009 until U11 returns to service.
7.2 A teletype sent on 14 October requests BON project change PH1 unit priority as follows: 1, 10, 9, 3,7,2,6,4,5,8, effective immediately until further notice. This moves Unit 9 to third in priority, and replaces the teletype sent on 29 July that moved Unit 7 to third in priority. This operation is necessary to complete a 100 day operation per PH1 rehab contract.

7.3 A teletype sent on 25 October requests BON to restrict outflows to 70 to 75 kcfs, hard constraint, between 1300 and 1600 on 26 October.  This operation was requested to move a boat house into the tailrace.
7.4 A teletype sent on 25 October requests BON to shift one unit of flow from PH2 to PH1 from 0700 until 1630 on 26 October.  This operation was requested is to support removal of hydrophone clusters in the PH2 forebay by Battelle.
7.5 A teletype sent on 26 October requests the following BON tailwater elevations for chum spawning:

7.5.1 Effective at 0600 on 27 October,  hard constraint of no less than 9.5’ msl during all hours
7.5.2 Effective at 0630 on 01 November until further notice, hard constraint of no less than 11.3’ msl during all hours, and a hard constraint of 11.3’ to 11.7’ with a target of 11.5’, from 0630 until 1700 daily. 

7.5.3 If needed, BON is authorized to pass water in excess of what is necessary to meet the 11.3’ to 11.7’ hard constraint by increasing tailwater elevation to 18.5’ msl between 1700 and 0600 hours.  Normal ramp rates apply.
7.5.4 If the actions in 7.5.3 are not sufficient, tailwater elevations may increase, as necessary, up to 12.5’ msl between 0600 and 1700 hours, daily.

7.5.5 The official tailwater gauge is located at river mile 144.5, on the Oregon shore.
7.6 A teletype sent on 27 October requests BON take PH1 units 6-10 out of service on 31 October from 1200 until 1700.  If needed, BON will spill to provide the conditions outlined in section 7.5.  This operation was requested to move a work barge for the JBS outfall removal into the tailrace.

7.7 A teletype sent on 28 October requests the following PH1 operations for outfall pipe removal:
7.7.1 Beginning at 0700 to 1700 each day from 01-08 November, operation of PH1 units is prohibited except as outlined below and Unit 0.
7.7.2 At 0700 on 01 November, Units 6-10 will be OOS.  At 0900 Units 1-5 will be OOS in preparation for dive activities.  Dive safety will determine if Units 3-6 can be brought on and still maintain safe diving conditions, beginning with Unit 3.

7.7.3 From 01-07 November, dive safety will contact BON control room daily and indicate which PH1 units are safe to operate. 

7.7.4 Units 3-6 will return to service each day when dive operations are complete.

7.7.5 To maintain BON’s tailwater elevation for chum (see 7.5), it may be necessary to spill.  All PH2 units should be loaded to maximum hydraulic capacity (outside 1% range) prior to spilling.
GLEN A. SMITH, P.E.
Operations Project Manager
Page 1 of 1
1
Page 3 of 3

