CENWP-OD-B





















05 June 2011
MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist
SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 22 of 2011, which covers the period from 29 May through 04 June 2011.
1 OPERATION SUMMARY

1.1 Daily average river flows ranged from 492.8 to 506.1 kcfs.  Daily average project tailwater ranged from 32.0’ to 32.5’ msl.  Daily average powerhouse forebay elevation ranged from 72.1’ to 73.5’ msl.  Daily average river temperature ranged from 53 to 56ºF. Secchi disk measurements ranged from 2.0’ to 3.0’.
1.2 Spill Bays 1 and 18, which provide attraction flow for Cascades Island and B-branch, respectively, are open 6” during daylight hours.  Daily average spill ranged from 282.7 to 293.3 kcfs.
1.3 Unit Operation.  Powerhouse One remains the priority powerhouse.
1.3.1 Unit 11 has remained out of service since 12 June 2008 due to rotor damage.

1.3.2 Unit 18 was out of service from 0704 until 0958 on 02 June for turbine bearing oil change and filter/strainer cleaning. 
1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris.  This week required fish units to be placed on standby to float debris during daylight hours due to excessive drawdown of up to 10-12 ft.  
1.5 Gatewell drawdowns are measured as needed and on Wednesdays.
2 ACTIVITIES

2.1 Auxiliary Water System Closures.  The following AWS valves were closed this reporting week:
2.1.1 FV 3-7 was closed for cleaning from 0715 until 0803 on 31 May.
2.1.2 FV 3-9 was closed for cleaning from 0658 until 0715 on 31 May.

2.2 STS/VBS inspections.  No STS/VBS inspections occurred this reporting week.
Table 1.  STS and VBS inspections.
	Unit
	Date
	Run Hours
	Comments

	11
	NA
	OOS
	U11 remains out of service.

	12-18
	NA
	NA
	Inspection scheduled for week of 27 June.


2.3 Dewatering and Fish Salvage.  No fish salvage activities occurred this reporting week.
3 RESEARCH  
3.1 Battelle - survival study.  Acoustic tagging and spring fish releases continue until 31 May.   
3.2 CRITFC – AFF.  Annual chinook, coho, sockeye and steelhead sampling in the AFF continues.
3.3 CRITFC - Sea lion hazing.  Hazing from boats has ended for the season.  Acoustic receivers for sea lion tags remain anchored in the BRZ.  
3.4 FFU - Sea lion.  Daily observations concluded 27 May.  Weekly pinniped reports can be found at

http://www.nwd-wc.usace.army.mil/tmt/documents/fish/2011/sea_lion_hazing2011.html
3.5 Kintama Research.  Acoustic tagging yearling Chinook at the SMF concluded this week.  
3.6 NOAA - COE Walla Walla District transportation evaluation.  Sort by Code collections at the SMF continue.
3.7 NOAA/PSMFC - lamprey.  LPS count network remains installed.  Traps deployed at PH2 monoliths in tailrace and AFF. 
3.8 PSMFC – Smolt Monitoring Program.  Juvenile fish condition and index monitoring continues.  Fallbacks recorded crossing the separator bars:  10 steelhead, 15 salmon and 29 unknown salmonids.  Gas Bubble Trauma (GBT) examinations occur two days per week at the SMF during spill.  Please go to http://www.fpc.org/CurrentDaily/Gbtsum.txt for further details.  A total of 25 yearling Chinook and 25 coho were collected for NOAA research this week.  
3.9 WDFW - Fish Counting.  Visual counting continues.
3.10 USDA - Sea lion and avian hazing. Hazing from the deck concluded.
4 FISHWAYS 
4.1 Project biologists inspected 29 May through 04 June.
4.2 Fish Passage Plan Observations.
5 Table 2.  FPP Out of Criteria items.
	DATE
	LOCATion
	FPP itEMS
	CAUSE
	RESPONSE
	TR#

	BRADFORD ISLAND (BI) 

	5/30
	B-branch entrance
	Differential < 1.0.’
	High tailwater, entrance elev. sensor
	Operations adjusted.
	

	CASCADES ISLAND (CI) – Nothing to report.

	WASHINGTON SHORE (WS)

	All week
	PH2 entrances
	Submergence <13’
	Necessary to maintain differential
	
	

	6/04
	DSM2 orifice 13C-N
	Bad prox switch
	Unknown
	TR.
	11-173098


5.1.1 PH1 PCC diffuser FG2-19 is partially open.  Repairs scheduled for 2011/2012 in water work period.  

5.1.2 A new mechanical velocity meter has been fabricated for PH2 and is scheduled for installation.
5.1.3 The SA-24 board that controls the Powerhouse 2 adult ladder components is in need of replacement. Work has begun on replacing the PLC to allow for fully automated operation of the PH2 adult fishway.
5.2 Fishway Temperatures.  Temperatures are taken 01 March through 30 November.
Table 3.  Bonneville Fishway Temperatures

	Date
	WA. Shore average
	BI

average

	5/29
	53.2
	53.7

	5/30
	53.6
	54.1

	5/31
	54.0
	54.4

	6/01
	53.9
	54.4

	6/02
	54.0
	54.5

	6/03
	54.3
	54.8

	6/04
	54.7
	55.2


5.3 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.  
5.4 Adult Fishways. 
5.4.1 Calibration was checked on 03 June.
5.4.2 10 of 12 floating orifice gates (FOGs) are installed with sea lion barriers in place.  Two gates remain out for maintenance. Due to high tailwater, water is flowing over the FOGs and bulkheads.
5.4.3 SLEDs remain installed at all fishway entrances.
5.4.4 The AFF remains watered up.
5.5 Juvenile Fishways.
5.5.1 DSM2 remains in service. 
5.5.2 All STSs are currently pulled.
5.5.3 The PH1 ice and trash sluiceway (ITS) remains in service.  Manual chaingates 1A and 1B remain open and are both set to 70’ msl. Automated chaingates 3B, 6C and 10B are open and functioning in auto.
5.5.4 The hydrocannon was turned off as a precautionary measure against high tail water.
5.5.5 The B2CC remains in service.  Five avian lines are broken due to high tailwater and three remain.  
5.6 Lamprey Fishways.
5.6.1 The Cascades Is. entrance weir remains installed.  The CI LPS remains out of service.
5.6.2 The NDE ramp remains in the lowered position and is not currently operating.
5.6.3 WA Shore LPS was watered up and operational on 25 May. Bradford Island LPS was watered up and operational on 31 May.
6 CONSTRUCTION
6.1 PH1 Gantry 4.  New crane in August, 2011.
6.2 Spillway Gate Repair Pit. Clean-up of hazardous material and shoring of the walls is complete.  Load test scheduled to occur spring 2011.
6.3 B2CC Hoist. Construction scheduled to begin fall of 2011.
7 HAZMAT, SPILLS AND CLEANUP. Nothing to report.
8 TELETYPES AND OTHER NOTEWORTHY ITEMS
8.1 A teletype sent on 19 May requests that BON operate Powerhouse One as the priority powerhouse. Continue this operation until further notice. The goal of this operation is to minimize debris loading that is contributing to increased descaling and mortality of juvenile fish at Powerhouse Two (PH2). Additionally, operate units 14, 15, and 16 without Submersible Traveling Screens. Vertical Barrier Screens (VBSs) will remain installed for these units until debris loading causes a drawdown of 1.5 ft. Once VBSs satisfy this criterion, remove VBSs in units 14, 15, and 16. When selecting operating range within the 1% range of the PH2 screened units (12, 13, 17, and 18) make efforts to the extent practicable to achieve the following two objectives: 1) Do not exceed the 1.5 ft head differential prior to the next cleaning and 2) Maximize discharge at each unit. Objective 1 takes priority over objective 2.
8.2 A teletype sent on 31 May, effective as early as system conditions allow, requests that BON make best efforts to maintain outflows near 510 kcfs, with a range of 505 kcfs to 515 kcfs. The goal is to not exceed a maximum instantaneous flow of 515 kcfs.
8.3 A teletype sent 03 June updated a former teletype requesting 100 kcfs spill from 10 April to 20 June for juvenile fish passage. The update clarifies that at no time is BON to voluntarily spill more than the fish passage spill cap on the revised TDG production estimate. 
JERRY CARROLL
Operations Project Manager
Page 1 of 3
1
Page 2 of 3

