CENWP-OD-B





















25 March 2012
MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist
SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 12 of 2012, which covers the period from 18 through 24 March 2012.
1 OPERATION SUMMARY

1.1 Daily average river flows ranged from 201.3 to 255.1 kcfs.  Daily average project tailwater ranged from 18.5’ to 20.7’ msl.  Daily average powerhouse forebay elevation ranged from 73.5’ to 75.1’ msl.  Daily average river temperature ranged from at 42 to 43ºF. Secchi disk measurements ranged from 4.5’ to 6.0’.
1.2 Spill Bays 1 and 18 provide attraction flow for Cascades Island and B-branch, respectively. Spill bays remain 6” open during daylight hours. Daily average spill ranged from 1.2 to 1.3 kcfs.
1.3 Unit Operation.  Powerhouse Two is the priority powerhouse.
1.3.1 Unit 11 has remained out of service since 12 June 2008 due to rotor damage.
1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris. 
1.5 Gatewell drawdowns are measured on Tuesdays and more frequently as needed.
2 ACTIVITIES
2.1 Auxiliary Water System Closures.  The following AWS closure occurred this reporting week.
2.1.1 FV 1-1 was closed for cleaning from 1633 until 1645 on 20 March.
2.2 STS/VBS inspections.  No STS/VBS inspection occurred this reporting week. 
2.2.1 Table 1.  STS and VBS inspection.  
	Unit
	Date
	Run Hours
	Comments

	11
	NA
	OOS
	U11 remains out of service.

	12-18
	NA
	NA
	Inspection scheduled for week of 01 April


2.3 Dewatering and Fish Salvage. No dewatering or fish salvage activities occurred this week.
3 RESEARCH    
3.1 Battelle - survival study.  Surgeon/tagging training and fish condition research continue at the JMF. Fish for training currently held in raceways at the SMF. 
3.2 CRITFC - Sea lion hazing.  Hazing from boats began on 05 March.
3.3 FFU - Sea lion.  Observations began on 06 January. Weekly pinniped reports can be found at: http://www.nwd-wc.usace.army.mil/tmt/documents/fish/2012/sea_lion_hazing2012.html
3.4 ODFW/WDFW - Sea lion. Sea lion traps remain adjacent to the B2CC and downstream of the old Navlock.  Acoustic tagging and branding began on 06 February. NOAA Fisheries authorized permanent removal of specific California sea lions effective 20 March. 
3.5 PSMFC – Smolt Monitoring Program.  Juvenile fish condition and index monitoring began on 01 March. Fallbacks recorded crossing the separator bars this week: 17 steelhead and 1 unknown salmonid. Fallback observations are available at the FPC website: http://www.fpc.org/adultsalmon/bonkeltcounts.htm
3.6 USDA - Sea lion hazing. Hazing from the deck began on 01 March.
3.7 WDFW - Fish Counting.  Video counting continues. Visual counting scheduled to begin on 01 April.
4 FISHWAYS 
4.1 Project biologists inspected 18 through 24 March.
4.2 Fish Passage Plan Observations.
4.2.1 Table 2.  FPP Out of Criteria items.
	DATE
	LOCATion
	FPP itEMS
	CAUSE
	RESPONSE
	TR#

	BRADFORD ISLAND (BI)

	03/20
	B-branch
	Entrance diff > 2.0’
	Unknown
	N/A
	N/A

	CASCADES ISLAND (CI) – Nothing to report.

	WASHINGTON SHORE (WS) – Nothing to report.


4.2.2 PH1CC diffuser FG2-19 is partially open.  Repairs will occur when the PH1CC is dewatered. The next possible time period will be 2013/14 winter maintenance.  

4.2.3 A new mechanical velocity meter has been fabricated for PH2 and is scheduled for installation.
4.3 Fishway Temperatures.  Temperatures are taken 01 March through 30 November. Temperatures are unavailable this week due to technical difficulties.
4.4 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.  
4.5 Adult Fishways. 
4.5.1 Calibration was checked on 24 March.
4.5.2 10 of 12 floating orifice gates (FOGs) are installed. Two gates remain out for maintenance. Stab plates were installed on 07 and 08 December in accordance with the FPP for 1 Fish Unit operations. Stab plates are scheduled for removal starting the week of 01 April.
4.5.3 All SLEDs are installed.
4.5.4 All ladders remain in service.

4.5.5 The AFF remains dewatered.

4.6 Juvenile Fishways.
4.6.1 DSM2 remains in service.
4.6.2 The PH1 ice and trash sluiceway (ITS) remains in service. Manual chaingates 1A and 1B remain open and are both set to 70’ msl. Automated chaingates 3B, 6C and 10B are open and functioning in auto. 
4.6.2.1 The ITS was closed from 0615 until 1318 on 21 March for hydrophone deployment in the PH1 forebay.
4.6.3 The hydrocannon remains in service.  
4.6.4 The B2CC was opened at 0718 on 19 March upon reaching the kelt trigger.
4.6.5 All B2CC and Spillway avian lines were removed to facilitate barge access for B-branch emergency repairs. The spillway array will be reinstalled the week of 26 March.
4.7 Lamprey Fishways.
4.7.1 The Cascades Is. entrance weir remains installed.
4.7.2 All LPS systems remain out of service for the season.
5 CONSTRUCTION
5.1 B2CC Hoist. Construction is complete. Final testing and modifications continue.
5.2 Seawater Recirculation System Removal (JMF). Removal continues.
6 HAZMAT, SPILLS AND CLEANUP. Nothing to report.
7 TELETYPES AND OTHER NOTEWORTHY ITEMS

7.1 A teletype sent on 23 November states in order to keep the latest established Bon tailwater flow range for chum, total project flow may be decreased to as low as 58 kcfs as long as the estimated flow at the confluence of the Columbia and Willamette rivers is equal to or exceeds 77 kcfs.  The 58 kcfs and 77 kcfs is a hard constraint navigation requirement.  

7.2 A teletype sent on 28 December replaces a previous teletype for operation of BON tailwater to provide adequate spawning conditions for Chum salmon. As a hard constraint, maintain tailwater at a minimum of 12.0’ all hours.
7.3 A teletype sent on 23 March undates the spill priority list order for the Columbia basin projects with associated total dissolved gas spill caps.

7.4 A teletype sent on 21 March requests that BON forebay be kept in the range of 75.0 – 76.5 feet, hard constraint, and 75.5 - 76.5 feet, soft constraint on Saturday, March 24 2012 from 0900 to 1200. The high forebay will allow divers to remove rocks and debris under a boat dock and ramp at the Port of Hood River.
7.5 A teletype sent on 23 March outlines specific steps of an operation intended to remove debris from the spillway apron. This operation will occur from 26 March until 02 April with periods of spill followed by a survey to determine of the spill had the desired effects. 
7.6 A teletype sent on 23 March states that the BON spillway will be out of service on 27 March from 1200 until 2000 hours and on 28 March from 0700 until 2000 hours. This outage provides safe tailrace conditions for contractors that will be working in the tailrace replacing avian lines.
JERRY CARROLL
Operations Project Manager
Page 1 of 3
1
Page 3 of 3

