CENWP-OD-B





















29 July 2012
MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist
SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 30 of 2012, which covers the period from 22 through 28 July 2012.
1 OPERATION SUMMARY

1.1 Daily average river flows ranged from 258.1 to 334.0 kcfs.  Daily average project tailwater ranged from 20.4’ to 24.5’ msl.  Daily average powerhouse forebay elevation ranged from 73.3’ to 74.2’ msl.  Daily average river temperature ranged from 67 to 68ºF. Secchi disk measurements ranged from 4.0’ to 5.2’.
1.2 Summer Spill for fish passage is in effect. Daily average spill ranged from 88.3 to 159.1 kcfs.
1.3 Unit Operation.  Powerhouse Two is the priority powerhouse.
1.3.1 Unit 2 was out of service from 0011 on 23 July until 1045 on 26 July for annual maintenance.

1.3.2 Unit 3 was out of service from 1001 to 1609 on 24 July and 0040 to 0140 on 25 July for substation maintenance. 
1.3.3 Unit 4 was out of service from 0954 on 24 July until 0141 on 25 July for substation maintenance.

1.3.4 Unit 5 was out of service from 1014 on 24 July until 0144 on 25 July for substation maintenance. 

1.3.5 Unit 6 was out of service from 0954 on 24 July until 0145 on 25 July for substation maintenance. 

1.3.6 Unit 7 was out of service from 1010 on 24 July until 0147 on 25 July for substation maintenance.
1.3.7 Unit 8 was out of service from 1010 on 24 July until 0147 on 25 July for substation maintenance. 

1.3.8 Unit 10 went out of service at 0928 on 09 July for a 5 year overhaul.
1.3.9 Unit 11 went out of service at 0005 on 06 July 2012 for governor trouble.

1.3.10 Unit 12 was out of service from 0707 to 1137 on 27 July for an STS inspection.

1.3.11 Unit 13 was out of service from 0657 to 0917 on 25 July for STS removal.
1.3.12 Unit 14 went out of service at 0710 on 11 June for a 4 year overhaul.
1.3.13 Unit 15 was out of service from 1126 to 1424 on 25 July for STS removal. 

1.3.14 Unit 16 was out of service from 0656 to 0950 on 26 July for STS removal.

1.3.15 Unit 17 was out of service from 0956 to 1218 on 26 July for STS removal.
1.3.16 Unit 18 was out of service from 1224 to 1702 on 26 July for STS removal.
1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris. 
1.5 Gatewell drawdowns are measured on Tuesdays and more frequently as needed.
2 ACTIVITIES
2.1 Auxiliary Water System Closures.  No AWS closures occurred this reporting week.
2.2 STS/VBS inspections.  No STS/VBS inspection occurred this reporting week. 
2.2.1 Table 1.  STS and VBS inspection.  

	Unit
	Date
	Run Hours
	Comments

	11,12,18
	7/25-27
	NA
	STSs inspected and repaired; remain installed

	13-17
	7/25-27
	NA
	STSs removed


2.3 Dewatering and Fish Salvage. 
2.3.1 On 24 July Project Biologists recovered 17 Pacific lamprey, 5 white sturgeon, 2 channel catfish, and 12 bullhead catfish from the Unit 11 draft tube. The fishes were released upstream in good condition. 
3 RESEARCH
3.1 Battelle/PNNL – Surgeon Training/Assessment. Surgeon assessment and Sort-By-Code activities will continue through July. 
3.2 Battelle/PNNL - Survival Study. Hydrophones remain installed.
3.3 CRITFC – AFF.  Annual Chinook, Coho, Sockeye and Steelhead sampling in the AFF continues.
3.4 NOAA - Lamprey. LPS count network installed. Trapping at the CI LCS, and the AFF continues.
3.5 NOAA – Pathology. 25 subyearling Chinook were collected at the JMF for research this week.
3.6 PSMFC – Smolt Monitoring Program.  Juvenile fish condition and index monitoring continues. Sort by Code is active for PNNL. Fallbacks recorded crossing the separator bars this week: 1 steelhead, 10 salmon, and 12 unknown salmonids. Fallback observations are available at the FPC website: http://www.fpc.org/adultsalmon/bonkeltcounts.htm. Gas Bubble Trauma (GBT) examinations occur two days per week at the SMF during spill.  Please go to http://www.fpc.org/CurrentDaily/Gbtsum.txt for further details. 
3.7 USDA - Avian hazing. Avian hazing is scheduled to conclude 30 July. 
3.8 University of Idaho. Cameras installed behind the Bradford Island picket leads to monitor lamprey and salmonid behavior. Lamprey trapping in conjunction with NOAA continues.
3.9 WDFW - Fish Counting.  Visual counting continues. Nighttime video lamprey counting continues.
4 FISHWAYS
4.1 Project biologists inspected 22 through 28 July.
4.2 Fish Passage Plan Observations.
4.2.1 Table 2.  FPP Out of Criteria items.
	Date
	Location
	FPP Items
	Cause
	Response

	SPILLWAY

	7/22-24
	Bay 17
	Incorrect height
	Hoist OOS
	None

	BRADFORD ISLAND (BI)

	7/22
	PH1CC N and S entrances
	Differentials < 1.0
	WG 1, 65 not controlling properly
	WGs moved to manual operation

	7/22
	PH1CC
	Velocity < 1.5
	Low entrance differentials
	Corrected with manual operation of WGs

	CASCADES ISLAND (CI) – Nothing to report.

	WASHINGTON SHORE (WS) – Nothing to report. 


4.2.2 PH1CC diffuser FG2-19 is partially open.  Repairs will occur when the PH1CC is dewatered. The next possible time period will be 2013/14 winter maintenance.  

4.2.3 A new mechanical velocity meter has been fabricated for PH2 and is scheduled for installation.
4.3 Fishway Temperatures.  Temperatures are taken 01 March through 30 November. BON site specific temperatures are unavailable this week due to technical difficulties. The temperatures below are provided via this website and show the averages for BON forebay.
	Date
	Temperature ((F)

	7/22
	66.0

	7/23
	65.9

	7/24
	66.2

	7/25
	66.8

	7/26
	67.4

	7/27
	67.0

	7/28
	66.8


4.4 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.  
4.5 Adult Fishways. 
4.5.1 Calibration was checked on 27 July.
4.5.2 10 of 12 floating orifice gates (FOGs) are installed. Two gates remain out for maintenance. 
4.5.3 PH1 and B-branch SLEDs were removed on 28 June. Cascades Island and WA shore SLEDs were removed on 16 July.
4.5.4 All ladders remain in service.
4.5.5 The AFF remains in service.
4.6 Juvenile Fishways.
4.6.1 DSM2 remains in service.
4.6.2 The PH1 ice and trash sluiceway (ITS) remains in service. Manual chaingates 1A and 1B remain open and are both set to 70’ msl. Automated chaingates 3B, 6C and 10B are open and functioning in auto.
4.6.3 The hydrocannon remains in service.
4.6.4 The B2CC remains open.
4.6.5 The B2CC avian array was removed to facilitate barge access for B-branch emergency repairs. The array will be reinstalled during winter of 2013.
4.7 Lamprey Fishways.
4.7.1 The Cascades Is. entrance weir remains installed.
4.7.2 All LPSs and associated traps remain in service.
5 CONSTRUCTION
5.1 Seawater Recirculation System Removal (JMF). Removal continues.
5.2 Cascades Island LPS Continuation. Construction on the exit section is scheduled to begin on 30 July. This expansion will remove the evaluation trap box and continue the LPS flume to a volitional exit near the CI fishway exit.
6 HAZMAT, SPILLS AND CLEANUP. Nothing to report.
7 TELETYPES CURRENTLY IN EFFECT
7.1 A teletype sent on 14 June states that, effective from 0001 hours on 16 June until 2400 hours on 31 August, operate in accordance with the 2012 Fish Operations Plan to provide spill for juvenile fish passage during the summer season at BON, as follows: 16 June – 21 July: Alternate between 2-day treatments of 95 kcfs 24 hours per day, and 85 kcfs day/121 kcfs night. 21 July – 31 August: 75 kcfs day/Gas cap night.
7.2 A teletype sent on 10 July requests that BON forebay be maintained between 73.0-74.0’ as a soft constraint from 1000 until 1700 on 21 July, and a 73.5’ minimum pool as a soft constraint from 1000 until 1700 on 22 July.

7.3 A teletype sent on 19 July updates the spill priority list order for Fish Passage Season for BON with associated total dissolved gas spill caps.

7.4 A teletype sent on 25 July requests BON forebay be maintained between 75.0–76.5’ as a hard constraint and 75.5-76.5’ soft constraint from 1400 to 2000 on 11 August for a sailboat race near The Dalles. 
JERRY CARROLL
Operations Project Manager
Page 1 of 3
1
Page 3 of 3

