CENWP-OD-B





















12 August 2012
MEMORANDUM FOR CENWP-OP, ATTN: Operations Biologist
SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for Week 32 of 2012, which covers the period from 05 through 11 August 2012.
1 OPERATION SUMMARY

1.1 Daily average river flows ranged from 194.1 to 230.3 kcfs.  Daily average project tailwater ranged from 16.3’ to 18.7’ msl.  Daily average powerhouse forebay elevation ranged from 73.0’ to 75.6’ msl.  Daily average water temperature ranged from 69 to 71ºF. Secchi disk measurements remained at 6.0’ all week.
1.2 Summer Spill for fish passage is in effect. Daily average spill ranged from 88.8 to 90.1 kcfs.
1.3 Unit Operation.  Powerhouse Two is the priority powerhouse.
1.3.1 Unit 10 went out of service at 0928 on 09 July for a 5 year overhaul.

1.3.2 Unit 11 went out of service at 0005 on 06 July 2012 for governor trouble.

1.3.3 Unit 12 went out of service at 2150 on 05 August for annual maintenance and T12 outage.

1.3.4 Unit 13 went out of service at 2151 on 05 August for T11 outage.

1.3.5 Unit 14 went out of service at 0710 on 11 June for a 4 year overhaul.
1.3.6 Unit 17 went out of service at 0735 on 01 August for air gap monitor installation.
1.4 Second Powerhouse fish units provide attraction flow for the Washington Shore (WS) fish ladder.  The units may be placed on standby, as necessary, during nighttime hours to float debris. 
1.4.1 Fish Unit 1 was out of service from 0741 until 1657 on 09 August for a ROV inspection.
1.4.2 Fish Unit 2 was out of service from 0743 until 1655 on 09 August for a ROV inspection.

1.5 Gatewell drawdowns are measured on Tuesdays and more frequently as needed.
2 ACTIVITIES
2.1 Auxiliary Water System Closures.  The following AWS closures occurred this reporting week.
2.1.1 FV 1-1 was closed from 0725 until 1500 on 07 August for a ROV inspection.
2.1.2 FV 3-7 was closed from 0725 until 1500 on 07 August for a ROV inspection.
2.1.3 FV 3-7 was closed from 0700 until 0720 on 06 August for cleaning.

2.1.4 FV 3-9 was closed from 0700 until 0720 on 06 August for cleaning.

2.1.5 FV 4-3 was closed from 0725 until 1130 on 08 August for a ROV inspection.
2.1.6 FV 4-4 was closed from 0725 until 1130 on 08 August for a ROV inspection.

2.1.7 FV 5-3 was closed from 1250 until 1600 on 08 August for a ROV inspection.

2.1.8 FV 5-4 was closed from 1250 until 1600 on 08 August for a ROV inspection.
2.2 STS/VBS inspections.  No STS/VBS inspection occurred this reporting week. 
2.2.1 Table 1.  STS and VBS inspection.  

	Unit
	Date
	Run Hours
	Comments

	11,12,18
	N/A
	N/A
	STSs remain installed

	13-17
	N/A
	N/A
	STSs removed


2.3 Dewatering and Fish Salvage. The following dewatering or fish salvage activities occurred this week.
2.3.1 On 06 August, the structural crew salvaged approximately 80 American shad and 1 carp from Tainter Valve 3. The fish were released into the Navlock.
3 RESEARCH
3.1 Battelle/PNNL – JMF Juvenile Assessment. Sort-By-Code activities concluded on 04 August. Juvenile monitoring may continue through September. 
3.2 Battelle/PNNL – Survival Study. Hydrophones remain installed.
3.3 CRITFC – AFF.  Annual Chinook, Coho, Sockeye and Steelhead sampling in the AFF continues.
3.4 NOAA – Lamprey. LPS count network installed. Trapping at the CI LTS and the AFF concluded on 08 August in anticipation of temperature restraints. Trapping may resume when temperatures allow.
3.5 NOAA – Pathology. Collection has concluded for the season.
3.6 PSMFC – Smolt Monitoring Program.  Juvenile fish condition and index monitoring continues. Sort by Code is active for PNNL. Fallbacks recorded crossing the separator bars this week: 1 steelhead, 2 salmon, and 2 unknown salmonids. Fallback observations are available at the FPC website: http://www.fpc.org/adultsalmon/bonkeltcounts.htm. Gas Bubble Trauma (GBT) examinations occur two days per week at the JMF during spill.  Please go to http://www.fpc.org/CurrentDaily/Gbtsum.txt for further details. 
3.7 University of Idaho. Camera monitoring behind the Bradford Island picket leads has concluded. Lamprey trapping in conjunction with NOAA may continue, but was halted on 08 August in anticipation of temperature restraints. Trapping may resume when temperatures allow.
3.8 WDFW – Fish Counting.  Visual counting continues. Nighttime video lamprey counting continues.
3.9 WDFW – Tule Counting. Scheduled to begin on 15 August.
4 FISHWAYS
4.1 Project biologists inspected 05 through 11 August. 
4.2 Fish Passage Plan Observations.
4.2.1 Table 2.  FPP Out of Criteria items.
	Date
	Location
	FPP Items
	Cause
	Response

	SPILLWAY – Nothing to report.

	BRADFORD ISLAND (BI)

	8/7
	PH1CC entrances
	No differential
	ROV inspection
	N/A

	8/8
	A-branch
	Staff gauge > 1.1’
	Unknown
	None

	8/8
	B-branch entrance
	No differential
	ROV inspection
	N/A

	8/8,10,11
	A-branch
	FG3-5 open w/<16.7 TW
	Burnt motor
	TR in place

	CASCADES ISLAND (CI)

	8/8
	Cascades Is. entrance
	No differential
	ROV inspection
	N/A

	WASHINGTON SHORE (WS) 

	8/9
	PH2CC entrances
	No differential
	ROV inspection
	N/A

	8/5, 9
	Weirs 37 and 38
	Water level < 0.9’
	W37 bleed-off incorrect position
	Ops adjusted

	8/5-11
	NDE
	Submergence < 13’
	Gate will not lower
	None


4.2.2 PH1CC diffuser FG2-19 is partially open.  Repairs will occur when the PH1CC is dewatered. The next possible time period will be 2013/14 winter maintenance.  

4.2.3 A new mechanical velocity meter has been fabricated for PH2 and is scheduled for installation.
4.3 Fishway Temperatures.  Temperatures are taken 01 March through 30 November. BON site specific temperatures are unavailable this week due to technical difficulties. The temperatures below are provided via this website and show the averages for BON forebay.
	Date
	Temperature ((F)

	8/05
	68.4

	8/06
	69.0

	8/07
	69.4

	8/08
	69.2

	8/09
	69.0

	8/10
	68.9

	8/11
	69.2


4.4 Zebra Mussel Monitoring.  No signs of colonization were observed this reporting week.  
4.5 Adult Fishways. 
4.5.1 Calibration was checked on 10 August.
4.5.2 10 of 12 floating orifice gates (FOGs) are installed. Two gates remain out for maintenance. 
4.5.3 SLEDs are removed from all entrances.
4.5.4 All ladders remain in service.
4.5.5 The AFF remains in service.
4.5.6 The Bradford Island exit was closed for cleaning from 0710 until 0720 on 06 August.
4.6 Juvenile Fishways.
4.6.1 DSM2 remains in service.
4.6.2 The PH1 ice and trash sluiceway (ITS) remains in service. Manual chaingates 1A and 1B remain open and are both set to 70’ msl. Automated chaingates 3B, 6C and 10B are open and functioning in auto.
4.6.3 The hydrocannon remains in service.
4.6.4 The B2CC remains open.
4.6.5 The B2CC avian array was removed to facilitate barge access for B-branch emergency repairs. The array will be reinstalled during winter of 2013.
4.7 Lamprey Fishways.
4.7.1 The Cascades Is. entrance weir remains installed.
4.7.2 All LPSs and associated traps remain in service except for the WA shore NDE trap.
5 CONSTRUCTION
5.1 Seawater Recirculation System Removal (JMF). Removal continues.
5.2 Cascades Island LPS Continuation. Construction on the exit section began on 30 July. This expansion will remove the evaluation trap box and continue the LPS flume to a volitional exit near the CI fishway exit.
5.3 Gantry 7 Rehabilitation. The Powerhouse 2 intake gantry crane went out of service on 01 August for rehabilitation. It is scheduled to return to service in February 2013.
6 HAZMAT, SPILLS AND CLEANUP. Nothing to report.
7 TELETYPES CURRENTLY IN EFFECT
7.1 A teletype sent on 14 June states that, effective from 0001 hours on 16 June until 2400 hours on 31 August, operate in accordance with the 2012 Fish Operations Plan to provide spill for juvenile fish passage during the summer season at BON, as follows: 16 June – 21 July: Alternate between 2-day treatments of 95 kcfs 24 hours per day, and 85 kcfs day/121 kcfs night. 21 July – 31 August: 75 kcfs day/Gas cap night.
7.2 A teletype sent on 25 July requests BON forebay be maintained between 75.0–76.5’ as a hard constraint and 75.5-76.5’ soft constraint from 1400 to 2000 on 11 August for a sailboat race near The Dalles. 
7.3 A teletype sent on 30 July states that BON project may operate PH2 unscreened units (Units 13-17) as necessary to pass flows. This operation is necessary due to the rehabilitation of Gantry 7. During the Gantry 7 outage, the project will not be able to remove or install screens. Additionally, limit flow through PH2 screened units (Units 11, 12, and 18) to a maximum of 15 kcfs per unit. The intent of this limitation is to minimize the change of damaging an operating screen during the Gantry 7 outage, which would force the unit out of service until February 2013.
7.4 A teletype sent on 09 August updates the spill priority list order for Fish Passage Season for BON with associated total dissolved gas spill caps.
JERRY CARROLL
Operations Project Manager
Page 1 of 3
1
Page 3 of 3

