CENWW-OD-EL HOLDREN 	                                                                       February, 2019

MEMORANDUM THRU:

Marty Mendiola, Operations Project Manager, Lower Granite Dam

FOR Chief, Operations Division

ATTN:   Eric Hockersmith / Ann Setter

SUBJECT:  Submission of 2018 Juvenile Fish Collection and Bypass Report, Lower Granite Dam Juvenile Fish Facility.

1.  Enclosed find the 2018 Juvenile Fish Collection and Bypass Report for Lower Granite Dam as requested.

2.  If you have any questions contact Elizabeth Holdren at Lower Granite Dam, (509) 843-2263.

ELIZABETH A. HOLDREN

Supervisory Fisheries Biologist, Lower Granite Dam

Enclosure

2018 Juvenile Fish Collection and Bypass Report

Lower Granite Dam Juvenile Fish Facility

Prepared by

Elizabeth Holdren

U.S. Army Corps of Engineers

and

Shawn Rapp Anchor QEA

February, 2019

TABLE OF CONTENTS

Page

Introduction	1

Facility Modifications	2

River Conditions	2

Fish Collection	3

Migration and Collection	3

Adult Fallbacks	5

Sampling	7

Transportation	8

Bypass	9

Incidental Species	10

Fish Condition	11

Descaling	11

Injuries and Disease	13

Mortality	14

Gas Bubble Trauma Monitoring (PSMFC)	17

Gatewell Dipping	16

Research	18

Operation and Maintenance	20

Turbine Operations	21

Debris/Trash Racks	22

Vertical Barrier Screens (VBSs)	22

Gatewells	22

Orifices/Collection Channel	22

Primary Dewaterer	23

Wet Separator/Distribution and Sampling Systems	23

Barge Loading Operations	23

Truck Loading Operations	23

Recommendations	23

LIST OF TABLES

Page

# Table 1.  Comparison of average monthly river flow and spill at LWG, 2012-2018.	3

# Table 2.  Annual collection, bypass, transportation and mortality at LWG, 2012-2018.	4

# Table 3.  Annual peak collection days at LWG, 2012-2018……………...……………………… 5

# Table 4.  Annual totals of adult salmonids released from the separator at LWG, 2012-2018.	5

# Table 5.  Monthly totals of adult salmonids released from the separator at LWG, 2018.	6

# Table 6.  Condition of adult salmonids released from the separator at LWG, 2018.	6

# Table 7.  Annual percentage of smolts sampled at LWG, 2012-2018.	8

# Table 8.  Weekly sample totals at LWG, 2018.	8

# Table 9.  Estimated collection of incidental fish species at LWG, 2018.	11

# Table 10.  Annual full-sample descaling rates (>20%) by species at LWG, 2012-2018.	12

# Table 11.  Weekly descaling rates in percent for fish sampled at LWG, 2018.	12

# Table 12.  Annual facility mortality in percent by species group at LWG, 2012-2018.	15

# Table 13.  Weekly facility mortality in percent by species group at LWG, 2018.	15

# Table 14.  Annual sample mortality by species group in percent at LWG, 2012-2018.	16

# Table 15.  Total barge mortalities from LWG 2012-2018.	17

# Table 16.  Annual truck mortality rate at LWG, 2012-2018.	17

Table 17.  Lower Granite turbine unit outages, 2018	22

LIST OF FIGURES

Page

Figure 1.  Daily average total flow and powerhouse flow at LWG, 2018.	3

Figure 2.  Fish collection and daily average flows at LWG, 2018	5

APPENDIX

Page

Appendix Table 1.  Daily collection and bypass numbers and river conditions at Lower Granite Dam, 2018.	1

Appendix Table 2.  Percent descaling and daily facility mortality numbers at Lower Granite Dam, 2018.	5

Appendix Table 3.  Daily number of fish trucked and barged from Lower Granite Dam, 2018.	9

Appendix Table 4.  Daily number of adult fallbacks and fallback mortality at Lower Granite Dam, 2018.	13

TRANSPORT OPERATIONS - LOWER GRANITE DAM

Introduction

The 2018 collection season at Lower Granite was characterized by average flow with the exception of May having higher than average flows.  Spring spill volume was determined by the total dissolved gas (TDG) cap as ordered by the court.  Water temperatures were slightly higher than average during April and September and below average the rest of the season.  Debris levels were high during the spring migration but low the rest of the season.  ESBSs (Extended Length Submersible Bar Screens) were installed March 19 through April 3.  The bypass system and juvenile collection facility were watered up in secondary bypass mode at 0700 hours April 1.  Operation of the JFF (Juvenile Fish Facility) in secondary bypass continued until collection for transport began at 0700 hours April 23.  Daily twenty-four hour collection for condition monitoring began at 0700 hours April 1 with the first sample worked up April 2.  Early season transport of Snake River anadromous salmonids as part of the ongoing study to compare in-river verses transported SARs (Smolt Adult Return Ratios) occurred April 12 and 19.  Four agencies conducted seven research projects and handled a total of 530,487 smolts at the Lower Granite juvenile collection facility this season in addition to smolt monitoring, gas bubble trauma (GBT) sampling, and kelt collection for Nez Perce Tribe reconditioning program.

Collection for transport began at 0700 hours April 23 and ended at 0700 hours October 31.  The facility was operated in primary bypass mode October 31 through November 13 when operations were changed to emergency bypass.  The juvenile bypass collection channel was dewatered November 15.  Total facility smolt collection for 2018 season was 6,473,432 fish compared to 5,430,419 in 2017.  Of the 6,473,432 fish collected during the 2018 season 1,345,283 were bypassed, 5,112,569 were barged, and 13,110 were trucked.

Pacific States Marine Fisheries Commission (PSMFC) technicians examined 948 fish for GBT between April 5 and June 7.  Smolts examined prior to April 23 were bypassed to the river and smolts examined after April 23 were transported.  There were 5 fish with symptoms of GBT observed during the 2018 season.

The passive integrated transponder (PIT) tag system detected 151,669 PIT-tagged fish at the JFF during the 2018 season.  Of the PIT tagged fish detected there were 74,973 diverted to the river, 72,752 diverted to the raceways for transport, 1,888 diverted to the sample holding tank, and 2,056 failed to be detected moving to bypass, raceways, or the sample.

Historically Snake River Basin hatchery salmonids were distinguished from wild salmonids by clipped adipose fins (occasionally pectoral or ventral fins).  Before 1998, Idaho Fish and Game (IDFG) was the only agency that released sizeable numbers of unclipped hatchery fish.  Starting in 1998, increasing numbers of unclipped hatchery fish were released by state, federal, tribal, or other agencies (such as the Fish Passage Center (FPC)).  Salmonids collected, sampled, bypassed, and transported from Lower Granite facilities are designated as clipped or unclipped not hatchery or wild.  Snake River Basin coho were reintroduced by the tribes and are all of hatchery origin.

Corps of Engineers personnel included: Project Supervisory Fisheries Biologist Elizabeth Holdren, Assistant Biologist Stephen Hampton, Maintenance Lead Jeremy Krewer, Lead Biological Technician Steven Lee; Biological Technicians: Bob Traufer, Marlene Fisher, Tyler Janazs, David Philips, Joshua Moore, Joe Parks, Kevin Ross, and Darin Hathaway; Biological Pathway Intern Sarah Gaulke; and Truck driver/maintenance personnel: Chuck Krasselt, Jeff Kuhn, and Tyler Potts.  Anchor QEA was represented by Biologists Shawn Rapp and Paul Burke. Representing Pacific States Marine Fisheries Commission (PSMFC) were Biologists Allan Martin and Jenna Davis.  Washington Department of Fish and Wildlife (WDF&W) was represented by Biologist Charles Morrill.  PSMFC technicians Cole Lampert, Praxy McIntyre, Jennifer Warner, and Garrett Nickum conducted fish sampling, and were responsible for the numerous quality control and data keeping tasks.

Facility Modifications

The following modifications were made to the JFF prior to or during the 2018 fish collection season:

- Juvenile bypass upgrade Phase 1a tie in to the existing facility was completed. Phase 1a construction/cleanup is ongoing.

- Refurbished the sample diversion slide gates per PSMFC guidelines.

- Sea chest and flapper seals replaced/installed on fish transport barge 8108.

- Continued replacing aerators biological balls on fish transport barges.

- Replaced facility supply manifold and lines.

- Continued replacing old mesh on raceway supply headbox screens to prevent fry and juvenile lamprey passage.

- Raised upstream raceway loading platform to uniform height for improved loading safety.

- Installed handles and securing pins to upstream raceway tailscreens eliminating the need for full removal.

- Fabricated and installed fueling platforms on barges 8105 and 8106.

- Raised fueling platforms on barges 8107 and 8108 series barges.

- Replaced/rebuilt all 48 hold water supply aerator valves on 8000 series barges.

River Conditions

The average daily river flow exceeded 100 kcfs on 49 days during the 2018 season.  Total river flows averaged 60.8 kcfs this season.  Highest daily average flow for the April 1-October 31 collection season was 180.4 kcfs May 27.  Lowest daily average flow for the season was 14.8 kcfs October 20.  Spilling through the RSW began at 0700 hours March 25 to provide fish passage due to Phase 1a construction and commissioning delaying JFF operations.  An emergency debris spill through the RSW occurred March 19.  Traditional spill for the fish passage occurred for 152 days from April 3 through midnight on August 31.  Spill is distributed according to FPP Table LWG-7 and LWG-8.  The seasonal average flow through spillways was 29.7 kcfs with a maximum daily average spill of 73.4 kcfs May 27 and a minimum daily average of 11.1 kcfs August 20.  The RSW was closed at 1420 hours August 8 due to total project outflow being less than 30 kcfs and flows forecasted to remain less than 30 kcfs for more than 3 consecutive days.  River temperatures collected as part of the daily condition sample averaged 59.4o F for the season and ranged from 43.5° F March 26 to 67.1° F July18.  A comparison of daily powerhouse flow and spill is shown in Figure 1.  Average monthly flow and spill for the 2014-2018 collection seasons are provided in Table 1.

Figure 1.  Daily average total flow and powerhouse flow at LWG, 2018.

Table 1.  Comparison of average monthly river flow and spill at LWG, 2014-2018.

| Flow (kcfs) | 2014 | 2015 | 2016 | 2017 | 2018 | 2014-17 Ave. |
| --- | --- | --- | --- | --- | --- | --- |
| April1 | 73.67 | 52.11 | 85.12 | 140.76 | 88.40 | 87.92 |
| May | 104.08 | 60.89 | 90.79 | 142.82 | 139.11 | 99.65 |
| June | 88.09 | 43.08 | 55.58 | 131.44 | 82.92 | 79.54 |
| July | 46.64 | 28.47 | 34.41 | 51.98 | 38.22 | 40.38 |
| August | 26.06 | 21.72 | 24.96 | 31.31 | 29.48 | 26.01 |
| September | 21.54 | 19.26 | 20.06 | 26.08 | 22.97 | 21.74 |
| October | 18.86 | 16.36 | 22.06 | 21.42 | 18.70 | 19.68 |
| Spill (kcfs) |  |  |  |  |  |  |
| April1 | 16.79 | 15.34 | 17.23 | 64.33 | 34.98 | 28.42 |
| May | 31.10 | 20.30 | 21.53 | 56.98 | 42.76 | 32.48 |
| June | 23.22 | 19.60 | 19.88 | 46.24 | 30.94 | 27.23 |
| July | 18.77 | 12.22 | 17.73 | 18.26 | 17.48 | 16.75 |
| August | 14.19 | 8.75 | 12.46 | 17.05 | 16.12 | 13.11 |
| September | 0.47 | 0.15 | 0.20 | 3.74 | 0.42 | 1.14 |
| October | 0.00 | 0.00 | 0.00 | 3.17 | 0.00 | 0.79 |

Fish Collection

##### Migration and Collection

Pre-transport secondary bypass for condition sampling collection did not occur from March 25-31 due to facility operations being delayed for Phase 1a construction and commissioning.  The RSW was put in operation to provide fish passage beginning at 0700 hours March 25.  Daily collection for condition sampling began April 1 and continued until transportation began at 0700 hours on May 1.  Collection for transport continued until 0700 hours October 31.  An estimated 6,473,432 juvenile salmonids were collected during the 2018 season compared to 5,430,419 in 2017 (Table 2).  Within each species group, the number collected, and percent of the total collection was: 2,342,198 clipped (36.2%) and 698,954 unclipped (10.8%) yearling Chinook, 208,584 clipped (3.2%), 329,159 unclipped (5.1%) subyearling Chinook, 1,877,057 clipped (29.0%) and 645,906 unclipped (10.0%) steelhead, 165,786 clipped (2.6%) and 22,959 unclipped (0.4%) sockeye/kokanee, and 182,829 coho (2.8%).  Daily collection and bypass numbers are provided in Appendix Table 1.

Table 2.  Annual collection, bypass, transportation and mortality at LWG, 2014-2018.

|  | Yearling Chinook |  | Subyearling Chinook |  | Steelhead |  | Sockeye/Kokanee |  | Coho |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | All | Total |
| Collection |  |  |  |  |  |  |  |  |  |  |
| 2014 | 2,431,937 | 1,010,410 | 242,870 | 415,002 | 1,856,040 | 548,219 | 18,902 | 112,122 | 52,724 | 6,688,226 |
| 2015 | 902,798 | 247,380 | 188,552 | 500,858 | 674,434 | 152,383 | 8,350 | 2,572 | 26,330 | 2,703,657 |
| 2016 | 3,405,400 | 1,104,727 | 262,101 | 512,157 | 2,385,586 | 600,528 | 28,700 | 4,665 | 150,416 | 8,454,280 |
| 2017 | 1,789,880 | 572,818 | 225,787 | 402,606 | 1,798,556 | 530,958 | 15,750 | 19,839 | 74,225 | 5,430,419 |
| 2018 | 2,342,198 | 698,954 | 208,584 | 329,159 | 1,877,057 | 645,906 | 165,786 | 22,959 | 182,829 | 6,473,432 |
| Bypass |  |  |  |  |  |  |  |  |  |  |
| 2014 | 895,654 | 605,721 | 3,013 | 8,991 | 920,228 | 156,884 | 0 | 59,640 | 3,722 | 2,653,853 |
| 2015 | 512,884 | 163,586 | 0 | 8,366 | 407,393 | 55,764 | 0 | 160 | 3,499 | 1,151,652 |
| 2016 | 2,263,433 | 842,120 | 596 | 31,941 | 1,564,652 | 311,329 | 258 | 655 | 33,079 | 5,048,063 |
| 2017 | 980,750 | 401,216 | 3,517 | 18,985 | 1,158,305 | 221,673 | 104 | 15,631 | 10,915 | 2,811,096 |
| 2018 | 353,084 | 257,945 | 1,075 | 13,690 | 599,397 | 112,054 | 12 | 4,483 | 3,543 | 1,345,283 |
| Truck |  |  |  |  |  |  |  |  |  |  |
| 2014 | 5 | 4 | 486 | 20,690 | 0 | 2 | 2 | 64 | 2 | 21,255 |
| 2015 | 28 | 9 | 145 | 22,184 | 28 | 16 | 0 | 7 | 15 | 22,432 |
| 2016 | 13 | 107 | 784 | 21,607 | 2 | 3 | 0 | 11 | 2 | 22,529 |
| 2017 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2018 | 0 | 7 | 665 | 12,387 | 0 | 2 | 0 | 40 | 9 | 13,110 |
| Barge |  |  |  |  |  |  |  |  |  |  |
| 2014 | 1,535,098 | 404,342 | 238,763 | 383,774 | 935,573 | 391,283 | 18,889 | 51,966 | 48,991 | 4,008,679 |
| 2015 | 389,616 | 83,675 | 188,023 | 468,810 | 266,752 | 96,530 | 8,091 | 2,392 | 22,805 | 1,526,694 |
| 2016 | 1,140,972 | 262,241 | 260,025 | 457,228 | 820,839 | 289,171 | 27,868 | 3,981 | 117,278 | 3,379,603 |
| 2017 | 807,461 | 171,227 | 220,591 | 380,436 | 640,117 | 309,241 | 15,613 | 4,086 | 63,247 | 2,612,019 |
| 2018 | 1,988,387 | 440,782 | 206,504 | 302,296 | 1,277,515 | 533,803 | 165,687 | 18,378 | 179,217 | 5,112,569 |
| Total Transport |  |  |  |  |  |  |  |  |  |  |
| 2014 | 1,535,103 | 404,346 | 239,249 | 404,464 | 935,573 | 391,285 | 18,891 | 52,030 | 48,993 | 4,029,934 |
| 2015 | 389,644 | 83,684 | 188,168 | 490,994 | 266,780 | 96,546 | 8,091 | 2,399 | 22,820 | 1,549,126 |
| 2016 | 1,140,985 | 262,348 | 260,809 | 478,835 | 820,841 | 289,174 | 27,868 | 3,992 | 117,280 | 3,402,132 |
| 2017 | 807,461 | 171,227 | 220,591 | 380,436 | 640,117 | 309,241 | 15,613 | 4,086 | 63,247 | 2,612,019 |
| 2018 | 1,988,387 | 440,789 | 207,169 | 314,683 | 1,277,515 | 533,805 | 165,687 | 18,418 | 179,226 | 5,125,679 |
| 2018 Mortalities |  |  |  |  |  |  |  |  |  |  |
| Facility | 727 | 220 | 340 | 786 | 145 | 47 | 87 | 58 | 60 | 2,470 |
| NMFS | 133 | 41 | 45 | 50 | 34 | 9 | 12 | 18 | 4 | 346 |
| Res/Sac | 6 | 36 | 9 | 16 | 19 | 16 | 0 | 0 | 0 | 102 |

1Includes Res/Sac fish and NMFS raceway mortalities.

By the end of May, 95.5% of the total yearly collection had occurred.  The percent of total collection arriving by the end of June and the end of July was 99.1% and 99.7%, respectively. The remaining 0.30% of juvenile salmonids was collected August through October.  The peak daily collection total and date for each species group were: clipped yearling Chinook 196,200 (May 9), unclipped yearling Chinook 33,600 (May 1), clipped subyearling Chinook 19,400 (May 29), unclipped subyearling Chinook 26,800 (May 27), clipped steelhead 93,403 (April 14), unclipped steelhead 49,400 (May 10), clipped sockeye 40,800 (May 17), unclipped sockeye/kokanee 1,600 (May 17), and coho 18,800 (May 10).  Total daily collection in 2018 peaked at 383,600 (May 10).  Daily collection of all species combined versus total flow is shown in Figure 2.Figure 2.  Peak collection date and daily collection total by species group are listed in Table 3.

Figure 2.  Fish collection and daily average flows at LWG, 2018.

Table 3.  Annual peak collection days at LWG, 2014-2018.

|  | Yearling Chinook |  | Subyearling Chinook |  | Steelhead |  | Sockeye/Kokanee |  | Coho |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | All | Total |
| 2014 | 6-May | 24-Apr | 3-Jun | 3-Jun | 27-Apr | 8-May | 18-May | 5-May | 19-May | 6-May |
|  | 287,000 | 48,000 | 12,900 | 18,600 | 126,200 | 31,000 | 8,900 | 10,800 | 5,600 | 438,800 |
| 2015 | 25-Apr | 25-Apr | 5-Jun | 5-Jun | 26-Apr | 7-May | 19-May | 9-May | 7-May | 26-Apr |
|  | 66,400 | 28,200 | 24,900 | 44,100 | 64,200 | 10,200 | 1,750 | 400 | 2,600 | 150,800 |
| 2016 | 26-Apr | 15-Apr | 10-Jun | 10-Jun | 24-Apr | 26-Apr | 20-May | 9-May | 9-May | 26-Apr |
|  | 208,800 | 99,046 | 18,100 | 25,900 | 197,000 | 52,000 | 4,600 | 400 | 21,200 | 492,000 |
| 2017 | 2-May | 15-Apr | 1-Jun | 29-May | 21-Apr | 9-May | 11-May | 17-Apr | 12-May | 21-Apr |
|  | 114,800 | 27,400 | 17,900 | 26,500 | 138,642 | 43,600 | 2,400 | 1,800 | 5,200 | 224,300 |
| 2018 | 9-May | 1-May | 29-May | 27-May | 14-Apr | 10-May | 17-May | 17-May | 10-May | 10-May |
|  | 196,200 | 33,600 | 19,400 | 26,800 | 93,403 | 49,400 | 40,800 | 1,600 | 18,800 | 383,600 |

##### Adult Fallbacks

A total of 2,425 adult salmonids fell back through the juvenile bypass system and were bypassed from the separator between April 1 and October 31 (Table 4).  The total includes 297 clipped and 350 unclipped adult chinook, 124 clipped and 252 unclipped jack Chinook, 762 clipped and 616 unclipped steelhead, 7 sockeye, and 17 Coho.  Steelhead kelts are included in the total though they are not technically fallbacks.  Fallbacks that went through the separator bars entered the raceways and were transported, sent through the sample system, or were secondary bypassed to the river were not counted by the separator technician.  No fallbacks were bypassed before April 1 or after October 31 due to the facility being in primary bypass and/or emergency bypass operation.  Daily adult fallbacks and fallback mortalities are in Appendix Table 4.

Table 4.  Annual totals of adult salmonids released from the separator at LWG (April 2-October 31) 2014-2018.

|  | Adult Chinook |  | Jack chinook |  | Steelhead |  | Sockeye |  | Coho |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | All | Totals |
| 2014 | 951 | 992 | 455 | 393 | 2,662 | 2,546 | 7 | 19 | 79 | 8,104 |
| 2015 | 558 | 483 | 290 | 201 | 1,180 | 1,472 | 9 | 7 | 5 | 4,205 |
| 2016 | 534 | 784 | 328 | 568 | 2,118 | 2,728 | 12 | 3 | 16 | 7,091 |
| 2017 | 91 | 101 | 19 | 16 | 611 | 820 | 1 | 0 | 0 | 1,659 |
| 2018 | 297 | 350 | 124 | 252 | 762 | 616 | 4 | 3 | 17 | 2,425 |
| 14-17 avg. | 534 | 590 | 273 | 295 | 1,643 | 1,891 | 7 | 7 | 25 | 5,265 |

Steelhead were the most common adult salmonid species removed from the separator in 2018.  April and May accounted for 84.5% of adult steelhead removed from the separator.  The remaining 15.5% of steelhead fallbacks were removed from the separator June through October.  The total number of steelhead fallbacks removed from the separator include out migrating kelts.  The majority of Chinook adults (78.1%) were removed from the separator during September and October (fall Chinook).  There were 4 clipped and 3 unclipped Sockeye fallbacks Sockeye fallbacks removed between July and September with 57.1% removed in September.  There were 17 coho fallbacks removed from the separator with 13 (76.5%) coho removed in October (Table 5).

Table 5.  Monthly totals of adult salmonids released from the separator at LWG (April 2-October 31), 2018.

|  | Adult Chinook |  | Jack Chinook |  | Steelhead |  | Sockeye |  | Coho |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | All | Totals |
| April1 | 0 | 0 | 0 | 0 | 159 | 101 | 0 | 0 | 0 | 260 |
| May | 34 | 7 | 1 | 0 | 527 | 378 | 0 | 0 | 0 | 947 |
| June | 35 | 34 | 2 | 2 | 22 | 97 | 0 | 0 | 0 | 192 |
| July | 8 | 16 | 2 | 0 | 1 | 1 | 1 | 0 | 0 | 29 |
| August | 2 | 6 | 0 | 1 | 6 | 4 | 1 | 1 | 0 | 21 |
| September | 23 | 39 | 9 | 43 | 19 | 15 | 2 | 2 | 4 | 156 |
| October | 195 | 248 | 110 | 206 | 28 | 20 | 0 | 0 | 13 | 820 |
| Totals | 297 | 350 | 124 | 252 | 762 | 616 | 4 | 3 | 17 | 2,425 |

1Includes March 26-31

Adult salmonid condition was classified as good, fair, poor, or dead prior to being released from the separator (Table 6).  Overall 92.5% of fallback condition was classified as good to fair.  Condition ratings of adult salmonids examined were as follows: 1,891 good (78.0%), 351 fair (14.5%), 140 poor (5.8%), and 43 mortalities (1.8%).  Adult salmonid mortalities included: 1 clipped and 1 unclipped Chinook, 30 clipped and 11 unclipped steelhead.  Adult Chinook had a higher percentage of good/fair fish (99.7%) than steelhead (86.9%).  A total of 84 adult lamprey were collected at the fish facility including 3 from the separator, 20 from raceways, 57 from the sample tank, 3 from the sample counting tank, and 1 from a raceway headbox.  Adult lamprey were transported and released into the Snake River upstream of Lower Granite Dam at Offield Landing boat ramp.

Table 6.  Condition of adult salmonids released from the separator at LWG (April 2-October 31), 2018.

|  | Adult Chinook |  | Jack chinook |  | Steelhead |  | Sockeye |  | Coho |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | All | Totals |
| Good | 273 | 321 | 121 | 246 | 450 | 460 | 2 | 2 | 16 | 1,891 |
| Fair | 23 | 28 | 3 | 6 | 184 | 104 | 1 | 1 | 1 | 351 |
| Poor | 0 | 0 | 0 | 0 | 98 | 41 | 1 | 0 | 0 | 140 |
| Dead | 1 | 1 | 0 | 0 | 30 | 11 | 0 | 0 | 0 | 43 |
| Total | 297 | 350 | 124 | 252 | 762 | 616 | 4 | 3 | 17 | 2,425 |

##### Sampling

Consistent with the 2018 Fish Operations Plan (FOP), Appendix B of the 2018 Fish Passage Plan, and guidance provided by the Regional Implementation Oversight Group (RIOG) through the Technical Management Team (TMT), the juvenile fish transportation program allows for a variable start date, based on data on fish survival, adult returns, current in-river conditions, and water supply forecasts.  All fish collected that are not needed for research will be bypassed to the river prior to the start of collection for transportation. This year TMT requested transportation collection at Lower Granite, Little Goose, and Lower Monumental to begin at 0700 hours April 23.  All fish sampled prior to transport were bypassed to the river with the exception of research fish.  Lower Granite fish sampling began at 0700 hours April 1 and ended at 0700 hours October 31.  Sampling at Lower Granite Dam is diverting and segregating groups of fish in a consistent fashion so that data collected from those segregated groups will accurately represent the sum total of the fish being collected in real time and is not the act of evaluating those groups.  A total of 211 daily samples were processed this season.  The sample rate was set at 10% April 1 and fluctuated from a minimum of 0.5% to a maximum of 100% based on guidelines provided by the Fish Passage Center (FPC), according to daily fish numbers, and to accommodate research needs.  During 2018, the smolt monitoring staff sampled 68,111 smolts or 1.1% of the total facility collection compared to 47,740 smolts (0.9% of the total facility collection) in 2017 (Table 7).

Table 7.  Annual percentage of smolts sampled at LWG, 2014-2018.

|  | Yearling Chinook |  | Subyearling Chinook |  | Steelhead |  | Sockeye/Kokanee |  | Coho |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | All | Total |
| 2014 | 0.6 | 0.8 | 2.9 | 8.5 | 0.6 | 0.6 | 0.8 | 1.1 | 0.5 | 1.2 |
| 2015 | 0.9 | 1.0 | 2.1 | 5.9 | 0.9 | 1.1 | 1.7 | 1.8 | 1.2 | 1.9 |
| 2016 | 0.6 | 0.6 | 2.2 | 6.9 | 0.6 | 0.6 | 1.2 | 1.0 | 0.6 | 1.0 |
| 2017 | 0.6 | 0.8 | 2.0 | 2.4 | 0.7 | 0.8 | 0.9 | 0.9 | 1.0 | 0.9 |
| 2018 | 0.6 | 0.7 | 3.1 | 7.9 | 0.6 | 0.6 | 0.5 | 1.1 | 0.6 | 1.1 |
| 14-17 Ave. | 0.6 | 0.7 | 2.3 | 6.0 | 0.6 | 0.7 | 1.1 | 1.0 | 0.7 | 1.1 |

The total number of smolts sampled in 2018 by species and percent of each species included: 13,235 clipped (19.4%) and 4,780 unclipped (7.0%) yearling Chinook, 6,459 clipped (9.5%) and 26,136 unclipped (38.4%) subyearling Chinook, 11,528 clipped (16.9%) and 3,736 unclipped (5.5%) steelhead, 839 clipped (1.2%) and 251 unclipped (0.4%) sockeye/kokanee, and 1,147 coho (1.7%) (Table 8).  The sample rate was raised above FPC guidelines April 24 to collect additional yearling Chinook and steelhead for the PNNL system survival study.

Table 8.  Weekly sample totals at LWG, 2018.

| Week Ending | Yearling Chinook |  | Subyearling Chinook |  | Steelhead |  | Sockeye/Kokanee |  | Coho |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | All | Total |
| 29-Mar | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 5-Apr | 717 | 815 | 0 | 9 | 637 | 33 | 0 | 71 | 0 | 2,282 |
| 12-Apr | 1,250 | 566 | 0 | 16 | 1,546 | 162 | 0 | 18 | 6 | 3,564 |
| 19-Apr | 699 | 636 | 0 | 43 | 1,545 | 272 | 0 | 5 | 11 | 3,211 |
| 26-Apr | 862 | 493 | 0 | 1 | 1,185 | 265 | 0 | 11 | 11 | 2,828 |
| 3-May | 3,261 | 737 | 0 | 8 | 2,074 | 610 | 0 | 12 | 108 | 6,810 |
| 10-May | 4,113 | 615 | 0 | 23 | 1,806 | 700 | 5 | 16 | 292 | 7,570 |
| 17-May | 1,822 | 444 | 21 | 79 | 1,146 | 672 | 579 | 37 | 199 | 4,999 |
| 24-May | 396 | 199 | 165 | 78 | 603 | 458 | 237 | 14 | 181 | 2,331 |
| 31-May | 60 | 108 | 647 | 842 | 510 | 286 | 10 | 14 | 122 | 2,599 |
| 7-Jun | 37 | 105 | 579 | 858 | 269 | 174 | 4 | 9 | 109 | 2,144 |
| 14-Jun | 11 | 37 | 844 | 1,172 | 139 | 60 | 3 | 1 | 55 | 2,322 |
| 21-Jun | 5 | 6 | 1,128 | 1,650 | 24 | 10 | 1 | 1 | 15 | 2,840 |
| 28-Jun | 1 | 6 | 616 | 1,306 | 28 | 6 | 0 | 0 | 17 | 1,980 |
| 5-Jul | 1 | 2 | 560 | 1,305 | 8 | 4 | 0 | 1 | 2 | 1,883 |
| 12-Jul | 0 | 3 | 478 | 1,360 | 2 | 0 | 0 | 0 | 3 | 1,846 |
| 19-Jul | 0 | 0 | 292 | 1,280 | 0 | 1 | 0 | 0 | 1 | 1,574 |
| 26-Jul | 0 | 0 | 208 | 1,476 | 0 | 1 | 0 | 0 | 2 | 1,687 |
| 2-Aug | 0 | 0 | 147 | 1,486 | 0 | 0 | 0 | 0 | 0 | 1,633 |
| 9-Aug | 0 | 0 | 85 | 863 | 2 | 1 | 0 | 0 | 0 | 951 |
| 16-Aug | 0 | 1 | 106 | 1,367 | 1 | 1 | 0 | 0 | 4 | 1,480 |
| 23-Aug | 0 | 0 | 176 | 2,413 | 1 | 0 | 0 | 0 | 0 | 2,590 |
| 30-Aug | 0 | 1 | 83 | 1,655 | 0 | 1 | 0 | 0 | 4 | 1,744 |
| 6-Sep | 0 | 0 | 22 | 343 | 0 | 2 | 0 | 1 | 0 | 368 |
| 13-Sep | 0 | 0 | 20 | 428 | 0 | 0 | 0 | 0 | 0 | 448 |
| 20-Sep | 0 | 0 | 6 | 169 | 0 | 2 | 0 | 1 | 2 | 180 |
| 27-Sep | 0 | 2 | 8 | 200 | 1 | 1 | 0 | 0 | 0 | 212 |
| 4-Oct | 0 | 1 | 10 | 152 | 0 | 1 | 0 | 5 | 0 | 169 |
| 11-Oct | 0 | 3 | 99 | 2,059 | 0 | 2 | 0 | 11 | 1 | 2,175 |
| 18-Oct | 0 | 0 | 58 | 1,208 | 1 | 3 | 0 | 3 | 0 | 1,273 |
| 25-Oct | 0 | 0 | 19 | 516 | 0 | 3 | 0 | 13 | 0 | 551 |
| 31-Oct | 0 | 0 | 82 | 1,771 | 0 | 5 | 0 | 7 | 2 | 1,867 |
| Total | 13,235 | 4,780 | 6,459 | 26,136 | 11,528 | 3,736 | 839 | 251 | 1,147 | 68,111 |

##### Transportation

An estimated 5,125,679 juvenile salmonids (79.1% of fish collected) were transported from Lower Granite Dam in 2018 by barge and truck combined.  The number of fish and the percentage of collection that was transported of each species group from Lower Granite included 1,988,387 clipped (84.9%) 440,789 unclipped (61.1%) yearling Chinook, 207,169 clipped (99.3%) and 314,683 unclipped (95.6%) subyearling Chinook, 1,277,515 clipped (68.1%) and 533,805 (82.6%) unclipped steelhead, 165,687 clipped (99.9%) and 18,418 unclipped (80.2%) sockeye/kokanee, and 179,226 coho (98.0%).  Early season transport occurred April 12 and April 19 as part of National Marine Fisheries Service (NMFS) study to compare transport and in-river yearling Chinook smolt to adult returns (SARs).  Daily barge transportation numbers are provided in Appendix Table 2.

Collection for barge transport occurred April 23 through August 14 from Lower Granite.  Every day barging operations occurred April 24 through May 22.  Every other day barging from Lower Granite occurred May 24 through August 14 with the exception of July 19.  The facility was in emergency bypass on July 18 and July 19 to repair a leak in the PDS to flume transition section.  Fish collection and transport from Little Goose and Lower Monumental dams continued as scheduled July 19.  Fish were released just above Ice Harbor dam July 3 due to a navigation lock outage at Ice Harbor Dam.  An estimated 5,112,569 juvenile salmonids (79.0% of fish collected) were transported by barge from Lower Granite Dam in 2018 including 1,988,387 clipped (84.9%) and 440,782 (63.1%) unclipped yearling Chinook, 206,504 clipped (99.0%) and 302,296 unclipped (91.8%) subyearling Chinook, 1,277,515 clipped (68.1%) and 533,803 unclipped (82.6%) steelhead, 165,687 clipped (99.9%) and 18,378 unclipped (80.0%) sockeye/kokanee, and 179,217 coho (98.0%).  Fish barged by Lower Granite staff as part of the barge trap and transport program from Lower Granite, Little Goose, and Lower Monumental during the 2018 season totaled 11,070,822 smolts.

Point Four oxygen monitoring systems were used on 4000 and 8000 series barges this season.  YSI portable oxygen monitoring units continue to be kept on barges as backup systems.  TDG probes were installed in one of the 2000, 4000, and 8000 series barges to monitor gas levels in barge holds as compared to river gas levels related to increasing spill levels to the gas cap.

Juvenile fish were trucked by pickup or semi-truck August 16 through October 31.  All truck trips were made with the 300 gallon pickup mounted midi-tank with the exception of October 9, 11, and 31 when fish pounds exceeded the midi-truck tanks capacity and the 3500 gallon semi-truck trailer was used.  Lower Granite transported 13,110 smolts by truck which is 0.2% of the total juvenile collection.  The number of smolts trucked by species included: 7 unclipped yearling Chinook, 665 clipped and 12,387 unclipped subyearling Chinook, 2 unclipped and 40 unclipped sockeye/kokanee, and 9 Coho.  Water temperatures and oxygen levels were monitored to ensure acceptable levels.  River water or river water ice is added to the truck transport tanks when needed to temper transport water in route and within 1-2⁰F of Bonneville tailrace.

##### Bypass

An estimated 1,345,283 juvenile salmonids were bypassed during the 2018 collection season.  The facility was operated in secondary bypass mode April 1 through April 23.  Bypassed fish were enumerated during the daily condition sample period from 0700 hours April 1 to 0700 hours April 23 an estimated 1,299,256 smolts or 20.1 % of the total 2018 season facility collection were bypassed.  During the April 24 through October 31 collection for transport season 46,027smolts were bypassed.  The total number bypassed during the collection season and percent of each species collected included 353,084 clipped (15.1%) and 257,945 unclipped (36.9%) yearling Chinook, 1,075 clipped (0.5%) and 13,690 unclipped (4.2%) subyearling Chinook, 599,397 clipped (31.9%) and 112,054 unclipped (17.3%) steelhead, 12 clipped (0.01%) and 4,483 unclipped (19.5%) sockeye/kokanee, and 3,543 coho (1.9%).  Facility bypass estimates include all fish bypassed to the tailrace during secondary bypass operation before collection for transport, GBT fish prior to April 24, and fish collected and provided for research needs.  Fish provided for research needs are recorded as bypassed including research mortalities.  There were 102 research mortalities reported during 2018 included 6 clipped and 36 unclipped yearling Chinook, 9 clipped and 16 unclipped subyearling Chinook, 19 clipped and 16 unclipped steelhead.  An additional 346 mortalities were removed from the east raceways that held NMFS transport research fish including 133 clipped and 41 unclipped yearling Chinook, 45 clipped and 50 unclipped subyearling Chinook, 34 clipped and 9 unclipped steelhead, 12 clipped and 18 unclipped sockeye/kokanee, and 4 Coho.  East raceway mortalities are included in Lower Granite facility mortality when raceways were also used for standard transport collection in addition to NMFS studies.  The facility bypassed fish estimate does not include fish bypassed by the PIT tag diversion system.  Juvenile salmonids were bypassed rather than transported for the following purposes this season.

- Secondary bypass occurred from April 1 through April 23.  Sampling occurred April 2 through April 23 for fish condition monitoring (COE).  Fish sampled during this period are included in the facility bypass total.  The facility was operated in primary  bypass from 0700 hours October 31 until 1033 hours November 13 when operations was changed to emergency bypass.  The collection channel was dewatered at 0930 hours November 15 for continued Phase 1 bypass system programing, construction, and repairs.  These fish are not included in the number of fish collected or bypassed this season.

- GBT inspections during the period of April 5 through April 19 accounted for a total of 310 fish bypassed. Within each species group the number bypassed was: 147 clipped yearling Chinook, 90 unclipped yearling Chinook, 57 clipped steelhead, and 16 unclipped steelhead.

- As part of research projects 55,123 fish were collected and bypassed (See; Research Section).  There were 1,371 fish mortalities removed from the east raceways used exclusively for research that were recorded as bypassed.  These fish are included in the bypass numbers of this report.

- The PTAGIS database revealed that 85,106 PIT-tagged fish of different species were bypassed through the PIT tag system.  These fish are not included in the facility bypass total.

According to the PTAGIS database, 120,561 PIT-tagged fish were detected at LWG in 2018.  Of the detected fish 85,106 (70.6%) were bypassed to the LWG tailrace through the PIT-tag diversion system, 34,259 (28.4%) were diverted to the raceways to be transported, 1,024 (0.9%) were diverted to the sample tank, and 2,056 (1.4%) were not detected again and their disposition was unknown.  From April 1 through April 23 all PIT-tagged fish were bypassed to the river.

##### Incidental Species

Non-target fish species that were too large to pass through the separator bars were recorded and bypassed through the adult release flume at the separator.  Incidental fish small enough to pass through the separator bars were either sampled and bypassed back to the river or held in raceways and transported with juvenile salmonids.  The number of incidental species counted in the daily sample were expanded based on the sample rate to calculate collection.  Incidental fish were added to the expanded collection from the sample to estimate the total collection for each incidental species.  An estimated 127,860 non-salmonid incidental fish were collected at the fish facility during the April 2 to October 31 passage period (Table 9).  This is 3.5 times more than the 36,215 incidental species collected in 2017 and the most since at least 2000.  Crappie, Longnose dace, Pacific Lamprey ammocoetes, sculpins, walleye, and Siberian prawns had their highest collection at the fish facility since at least the year 2000.  There were 42 white sturgeon observed this season which is the most since 2004.  Siberian Prawns were the most abundant incidental species in 2018 with an estimated 43,434 collected compared to 4,148 collected in 2017 and 25,848 collected in 2016. Siberian prawns were euthanized per Washington Department of Fish and Wildlife protocol and disposed of according to the Washington Department of Fish and Wildlife permit requirements.  Pacific lamprey macrophthalmia were the second most abundant incidental species with an estimated 40,736 collected.  Pacific lamprey ammocoetes were the third most abundant incidental species with estimated 10,212 collected.

Table 9.  Estimated collection of incidental fish species at LWG, 2018.

| Common Name | Scientific Name | Separator | Expanded Sample | Total Collection1 |
| --- | --- | --- | --- | --- |
| American Shad (Adult) | Alosa sapidissima | 182 | 56 | 238 |
| American Shad (Juvenile) | A. sapidissima |  | 2,155 | 2,155 |
| Banded Killifish | Fundulus diaphanus |  |  |  |
| Bass, Largemouth | Micropterus salmoides |  | 1 | 1 |
| Bass, Smallmouth | M. dolomieui | 7 | 3,618 | 3,625 |
| Bullhead (misc.) | Amierus sp. | 3 | 159 | 162 |
| Catfish, Channel | Ictalurus punctatus | 45 | 1,330 | 1,375 |
| Catfish, Flathead | Pylodictis olivaris | 1 |  | 1 |
| Chiselmouth | Acrocheilus alutaceus | 4 | 275 | 279 |
| Common Carp | Cyprinus carpio | 135 | 80 | 215 |
| Crappie (misc) | Pomoxis sp. | 33 | 9,926 | 9,959 |
| Dace, Longnose | Rhinichthys cataractae |  | 227 | 227 |
| Dace, Speckled | R. osculus |  |  |  |
| Kokanee2 | Oncorhynchus nerka |  | 1 | 1 |
| Northern Pikeminnow | Ptychocheilus oregonensis | 2 | 140 | 142 |
| Pacific Lamprey (Adult) | Entosphenus tridentatus | 3 | 83 | 86 |
| Pacific Lamprey (Ammocoete) | E. tridentatus |  | 10,212 | 10,212 |
| Pacific Lamprey (Macrophthalmia) | E. tridentatus |  | 40,736 | 40,736 |
| Peamouth | Mylocheilus caurinus | 41 | 1,346 | 1,387 |
| Redside Shiner | Richardsonius balteatus |  |  |  |
| Sand Roller | Percopsis transmontana | 4 | 3,460 | 3,464 |
| Sculpin | Cottus sp. |  | 477 | 477 |
| Siberian Prawn | Exopalaemon modestus |  | 43434 | 43,434 |
| Sucker (misc.) | Catostomus sp. | 617 | 3,663 | 4,280 |
| Sunfish (misc.) | Lepomis sp. | 8 | 1,265 | 1,273 |
| Trout, Bull | Salvelinus Malma | 3 |  | 3 |
| Trout, Cutthroat | Oncorhynchus clarkii |  |  |  |
| Trout, Rainbow | O. mykiss | 1003 | 2214 | 321 |
| Walleye | Stizostedion vitreum | 5 |  | 5 |
| Warmouth | Lepomis gulosis |  | 55 | 55 |
| White Sturgeon | Acipenser transmontanus | 42 |  | 42 |
| Whitefish | Prosopium sp. | 8 | 3,695 | 3,703 |
| Yellow Perch | Perca flavescens | 1 | 1 | 2 |
| Total |  | 1,244 | 126,616 | 127,860 |

1Separator count plus expanded sample count equals estimated total facility collection.

2Unclipped Oncorhynchus nerka not CWT or PIT-tagged and >200mm

3Large steelhead smolts that cannot fit through the narrower spaced separator bars.

4Steelhead lacking smoltification characteristics and/or fish under 140mm, per FPC guidelines.

Fish Condition

##### Descaling

The standard descaling criteria is classified as a fish with 20% or greater scale loss on one side of its body.  Scale loss less than 20% on one side of a fishes body is not considered descaled.  PSMFC smolt monitoring personnel collected descaling data from all live sample fish (full sample) rather than just a portion (subsample).

The descaling rate for all fish sampled in 2018 was 1.5% which is compared to the 2014-2017 average of 1.6% (Table 10).  The annual descaling rate by species group was: clipped yearling Chinook 1.3%, unclipped yearling Chinook 0.8%, clipped subyearling Chinook 0.6%, unclipped subyearling Chinook 0.8%, clipped steelhead 3.6%, unclipped steelhead 2.7%, clipped sockeye 1.0%, unclipped sockeye/kokanee 3.3%, and Coho 1.4%.

Table 10.  Annual full-sample descaling rates (>20%) by species at LWG, 2014-2018.

|  | Yearling Chinook |  | Subyearling Chinook |  | Steelhead |  | Sockeye/Kokanee |  | Coho |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | All | Total |
| 2014 | 0.9 | 0.6 | 0.7 | 1.8 | 1.3 | 1.2 | 0.0 | 2.6 | 1.0 | 1.3 |
| 2015 | 1.6 | 1.2 | 0.7 | 2.6 | 2.2 | 3.3 | 5.1 | 0.0 | 0.9 | 2.2 |
| 2016 | 0.9 | 0.5 | 1.0 | 2.0 | 1.1 | 1.3 | 1.2 | 2.2 | 1.1 | 1.4 |
| 2017 | 1.8 | 0.8 | 1.3 | 1.5 | 2.7 | 1.8 | 5.6 | 4.7 | 1.1 | 1.9 |
| 2018 | 1.3 | 0.8 | 0.6 | 0.8 | 3.6 | 2.7 | 1.0 | 3.3 | 1.4 | 1.5 |
| 2014–2017 Avg | 1.2 | 0.7 | 0.9 | 2.1 | 1.8 | 1.7 | 2.5 | 2.7 | 1.1 | 1.6 |

The highest weekly descaling rate for all species combined was 3.8% for the week ending May 24.  The highest weekly descaling rates are normally observed in late August, September, and October when temperatures increase, flows decrease, and the sample size decreases.  Heavy debris load early in the season likely contributed to increased descaling rates in May. The lowest descaling rates are generally during June and July when small subyearling Chinook salmon dominate the collection.  In 2018 descaling rates were lowest during June, July and August (Table 11).  Clipped sockeye collected at the juvenile fish facility in late May and early June exhibited caudal fin rot and fungus but not descaling like in previous years.  IDFG determined these maladies were due to hatchery related fish transport and release conditions and not Lower Granite operation.    Daily descaling rates are provided in Appendix, Table 3.

Table 11.  Weekly descaling rates in percent for fish sampled at LWG, 2018.

| Week Ending | Yearling Chinook |  | Subyearling Chinook |  | Steelhead |  | Sockeye/Kokanee |  | Coho |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | All | Total |
| 29-Mar | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |
| 5-Apr | 1.68% | 1.11% | -- | -- | 1.26% | 0.00% | -- | 1.41% | -- | 1.32% |
| 12-Apr | 0.40% | 0.89% | -- | -- | 0.91% | 1.85% | -- | 0.00% | 0.00% | 0.76% |
| 19-Apr | 1.30% | 0.47% | -- | -- | 1.23% | 1.49% | -- | 0.00% | 9.09% | 1.14% |
| 26-Apr | 1.28% | 0.41% | -- | -- | 3.04% | 1.89% | -- | 0.00% | 0.00% | 1.92% |
| 3-May | 1.26% | 1.09% | -- | -- | 3.39% | 2.79% | -- | 8.33% | 1.85% | 2.05% |
| 10-May | 1.37% | 0.16% | -- | 0.00% | 5.39% | 2.00% | 0.00% | 0.00% | 0.68% | 2.26% |
| 17-May | 1.71% | 0.45% | 0.00% | 0.00% | 5.95% | 3.13% | 0.52% | 2.70% | 2.01% | 2.63% |
| 24-May | 1.78% | 1.51% | 0.61% | 0.00% | 7.79% | 4.81% | 1.69% | 0.00% | 2.76% | 3.85% |
| 31-May | 0.00% | 1.85% | 0.00% | 0.24% | 6.68% | 2.80% | 0.00% | 15.38% | 0.82% | 1.90% |
| 7-Jun | 2.70% | 0.00% | 0.70% | 0.63% | 4.85% | 2.30% | 0.00% | 0.00% | 0.92% | 1.35% |
| 14-Jun | 0.00% | 2.70% | 0.24% | 0.18% | 2.88% | 3.33% | 50.00% | 0.00% | 0.00% | 0.53% |
| 21-Jun | 0.00% | 0.00% | 0.36% | 0.19% | 4.17% | 0.00% | 0.00% | 100.00% | 0.00% | 0.32% |
| 28-Jun | 0.00% | 16.67% | 0.49% | 0.39% | 0.00% | 0.00% | -- | -- | 0.00% | 0.46% |
| 5-Jul | 0.00% | 50.00% | 0.18% | 0.39% | 0.00% | 0.00% | -- | 0.00% | 0.00% | 0.38% |
| 12-Jul | -- | 33.33% | 0.84% | 0.74% | 0.00% | -- | -- | -- | 0.00% | 0.82% |
| 19-Jul | -- | -- | 0.70% | 1.27% | -- | 0.00% | -- | -- | 0.00% | 1.17% |
| 26-Jul | -- | -- | 0.00% | 0.41% | -- | 0.00% | -- | -- | 0.00% | 0.36% |
| 2-Aug | -- | -- | 0.72% | 0.55% | -- | -- | -- | -- | -- | 0.56% |
| 9-Aug | -- | -- | 1.20% | 0.94% | 0.00% | 0.00% | -- | -- | -- | 0.96% |
| 16-Aug | -- | 0.00% | 1.94% | 0.96% | 0.00% | 0.00% | -- | -- | 0.00% | 1.02% |
| 23-Aug | -- | -- | 1.14% | 0.54% | 0.00% | -- | -- | -- | -- | 0.58% |
| 30-Aug | -- | 0.00% | 0.00% | 0.91% | -- | -- | -- | -- | 0.00% | 0.87% |
| 6-Sep | -- | -- | 0.00% | 0.88% | -- | 0.00% | -- | 0.00% | -- | 0.82% |
| 13-Sep | -- | -- | 0.00% | 2.61% | -- | -- | -- | -- | -- | 2.49% |
| 20-Sep | -- | -- | 0.00% | 1.80% | -- | 0.00% | -- | 0.00% | 0.00% | 1.69% |
| 27-Sep | -- | 0.00% | 0.00% | 3.00% | 0.00% | 0.00% | -- | -- | -- | 2.83% |
| 4-Oct | -- | 0.00% | 0.00% | 3.31% | -- | 0.00% | -- | 0.00% | -- | 2.99% |
| 11-Oct | -- | 0.00% | 3.09% | 1.42% | -- | 0.00% | -- | 9.09% | 0.00% | 1.53% |
| 18-Oct | -- | -- | 3.45% | 1.41% | 0.00% | 0.00% | -- | 0.00% | -- | 1.50% |
| 25-Oct | -- | -- | 0.00% | 1.95% | -- | 0.00% | -- | 7.69% | -- | 2.00% |
| 31-Oct | -- | -- | 7.32% | 1.13% | -- | 0.00% | -- | 0.00% | 0.00% | 1.40% |
| # Descaled | 173 | 39 | 38 | 215 | 411 | 100 | 8 | 8 | 16 | 1,008 |
| # Sampled | 13,192 | 4,765 | 6,404 | 25,599 | 11,493 | 3,726 | 833 | 246 | 1,147 | 67,405 |
| % Descaled | 1.31% | 0.82% | 0.59% | 0.84% | 3.58% | 2.68% | 0.96% | 3.25% | 1.39% | 1.50% |

##### Injuries and Disease

Injury data was gathered from a sub sample of 100 of the dominant species and not more than 100 each of the non-dominant species.  There were 27,404 fish examined for injury and disease and 6,617 fish (24.1 %) were afflicted with an injury or disease symptom in 2018.  The overall affliction rate, body injury rate, predator injury rate, head injury rate and disease symptom rates reported are the actual rates observed for 2014 to 2018.  In previous years, these rates were reported with the caveat that the actual injury rates are lower than reported due to individual fish having more than one symptom or injury.  The body injuries associated with dam passage that were recorded this season included a generic body injury category and a generic fin injury category.  Head injuries that were associated with dam passage include generic head injuries, eye injuries, operculum injuries and “pop” or bulging eye.  Fish were also examined for external symptoms of fungus, columnaris, bacterial kidney disease, and parasites.

Body injuries were observed on 19.8% of the smolts examined in the detailed subsample.  Blood pooling is defined as the vasodilatation of the capillaries in fins (also referred to as fin pinkness).  It seems to be a symptom of anesthetic use during higher water temperatures and is mostly found on subyearling Chinook.  Fin hemorrhaging is the discharge of blood outside the body and is a sign of trauma.  Of the smolts examined from the sample that had body injuries, the most common symptom observed in 2018 was general fin injury (39.2%), followed by pink fin (35.6%), fin discoloration (21.2%), body injury (2.3%), and body deformities (1.6%).  Clipped sockeye exhibited the highest percent of body injuries at 44.5% (286 of 643 examined) followed by unclipped subyearling Chinook at 27.3% (2,986 of 10,931 examined).

Head injuries were recorded on 0.4% of the smolts examined in the detailed subsample. Unclipped sockeye/kokanee had the highest incidence of head injury at 2.1% (5 of 237 examined), followed by Coho at 0.7% (8 of 1,145 examined).  Injuries to the operculum comprised the majority of observed head injuries at 31.4%, followed by eye injuries at 25.4%, general head injuries at 21.2%, eye hemorrhage at 16.9 %, “pop” eye at 5.1%.

Injuries associated with predators include wounds inflicted by other fish, birds, and lamprey.  Predator wounds were observed on 1.0% of the smolts examined.  Predator marks caused by birds, characterized by a distinct V-shaped descaling pattern on both sides of a fish were the most common predator mark at 56.6% compared to 40.5% caused by fish and 2.9% caused by lamprey.  Predator marks were highest on clipped steelhead at 1.9% (89 of 4,806 examined), unclipped steelhead at 1.5% (26 of 1,707 examined), and clipped yearling Chinook at 1.4% (49 of 3,426 examined).

External symptoms of disease were observed on 5.9% of the smolts examined in the detailed subsample compared to 2.9% in 2017, 5.6% in 2016, 3.5% in 2015, and 3.3% in 2014.  Symptoms of disease were most common on unclipped subyearling Chinook (10.1%).  Of the fish afflicted, fin hemorrhages comprised the majority of disease symptoms (75.3%), followed by columnaris (12.7%), parasites (6.3%), fungus (3.2%), and bacterial kidney disease (2.6%).

Fin hemorrhage was found on 4.5% of all fish examined and on all species and rearing types.  Fin hemorrhage was the primary disease afflicting unclipped subyearling Chinook and was observed on 8.1% of unclipped subyearling Chinook examined.

Columnaris is caused by the bacterium Flavobacterium columnare that becomes more virulent as water temperatures increase.  Summer and subyearling migrants are more susceptible to infection as water temperatures increase during their outmigration.  Columnaris can be recognized by the presence of yellowish lesions on the belly, damage to the gills, pelvic fins, snout, and caudal fins.  SMP at Lower Granite only classify fish as being infected with columnaris if there is some tissue loss on the snout or body (fish with only red mouth edges are not classified as infected).  Columnaris was identified at Lower Granite in 1996 and symptoms observed on sample fish have been recorded since 1999.  Typically, the first incidence of columnaris is observed in July after the majority of subyearling Chinook have passed the Project.  The first symptoms of columnaris this season were observed July 8.  The 2018 columnaris infection rate for subyearling Chinook was 1.0% (319 of 31,986) compared to 2014-2017 average of 1.3%.

##### Mortality

Facility mortality includes fish removed from the barges or trucks before departure, sample mortalities, recovery tank mortalities, separator mortalities and raceway mortalities, not including the east raceways when used to hold NMFS research fish.  Mortalities removed from east raceways when used exclusively for NMFS studies were included in bypassed fish and not considered facility mortalities.  Annual facility mortality for all groups combined was 0.04% in 2018 and totaled 2,470 fish (Table 12).  This is the lowest facility mortality rate since 1996. All species group mortality rates were lower than those observed for the 2014-2017 average except for clipped steelhead.  Within each species group the number of facility mortalities and percent of those collected in that group was: 727 clipped (0.03%) and 220 unclipped (0.03%) yearling Chinook, 340 clipped (0.16%) and 786 unclipped (0.24%) subyearling Chinook, 145 clipped (0.01%) and 47 unclipped (0.01%) steelhead, 87 clipped (0.05%) and 58 unclipped (0.25%) sockeye/kokanee, and 60 coho (0.03%).

Table 12.  Annual facility mortality in percent by species group at LWG, 2014-2018.

|  | Yearling Chinook |  | Subyearling Chinook |  | Steelhead |  | Sockeye/Kokanee |  | Coho |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | All | Total |
| 2014 | 0.05 | 0.03 | 0.25 | 0.37 | 0.01 | 0.01 | 0.06 | 0.40 | 0.02 | 0.06 |
| 2015 | 0.03 | 0.04 | 0.20 | 0.29 | 0.04 | 0.04 | 3.03 | 0.47 | 0.04 | 0.10 |
| 2016 | 0.03 | 0.02 | 0.27 | 0.27 | <0.01 | <0.01 | 2.00 | 0.39 | 0.04 | 0.05 |
| 2017 | 0.09 | 0.07 | 0.74 | 0.79 | 0.01 | 0.01 | 0.21 | 0.61 | 0.08 | 0.13 |
| 2018 | 0.03 | 0.03 | 0.16 | 0.24 | 0.01 | 0.01 | 0.05 | 0.25 | 0.03 | 0.04 |
| 14-17 Ave. | 0.05 | 0.04 | 0.37 | 0.41 | 0.01 | 0.01 | 1.20 | 0.43 | 0.05 | 0.08 |

Weekly facility mortality rates were low during April and May and increased during June and July as water temperatures increased, river flows decreased, and the sample size decreased.  The weekly facility mortality rate increased to over 1.0% the week ending August 2, remained around 1% during August and September and decreased during October.  The maximum weekly mortality rate of 1.67% occurred the week ending September 20 when only 180 fish were collected (Table 13).

Table 13.  Weekly facility mortality in percent by species group at LWG, 2018.

| Week Ending | Yearling Chinook |  | Subyearling Chinook |  | Steelhead |  | Sockeye/Kokanee |  | Coho |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | All | Total |
| 29-Mar | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |
| 5-Apr | 0.01% | 0.01% | -- | 0.50% | 0.00% | 0.00% | -- | 0.00% | -- | 0.01% |
| 12-Apr | 0.00% | 0.00% | -- | 0.10% | 0.01% | 0.00% | -- | 0.00% | 0.00% | 0.00% |
| 19-Apr | 0.01% | 0.00% | -- | 0.02% | 0.00% | 0.01% | -- | 0.00% | 0.00% | 0.00% |
| 26-Apr | 0.04% | 0.05% | -- | 0.00% | 0.00% | 0.00% | -- | 0.69% | 0.11% | 0.02% |
| 3-May | 0.02% | 0.03% | -- | 0.00% | 0.01% | 0.00% | -- | 0.54% | 0.01% | 0.02% |
| 10-May | 0.03% | 0.04% | -- | 0.04% | 0.01% | 0.01% | 0.00% | 0.25% | 0.02% | 0.03% |
| 17-May | 0.05% | 0.04% | 0.14% | 0.22% | 0.01% | 0.01% | 0.05% | 0.16% | 0.04% | 0.04% |
| 24-May | 0.08% | 0.11% | 0.05% | 0.14% | 0.00% | 0.01% | 0.06% | 0.21% | 0.02% | 0.04% |
| 31-May | 0.18% | 0.11% | 0.16% | 0.14% | 0.01% | 0.02% | 0.00% | 0.40% | 0.07% | 0.11% |
| 7-Jun | 0.06% | 0.06% | 0.16% | 0.14% | 0.02% | 0.04% | 1.00% | 0.00% | 0.14% | 0.12% |
| 14-Jun | 0.67% | 0.09% | 0.13% | 0.11% | 0.20% | 0.12% | 1.33% | 0.00% | 0.12% | 0.13% |
| 21-Jun | 0.00% | 0.00% | 0.11% | 0.12% | 0.31% | 0.00% | 10.00% | 0.00% | 1.32% | 0.13% |
| 28-Jun | 0.00% | 0.00% | 0.20% | 0.26% | 0.00% | 0.00% | -- | -- | 0.00% | 0.24% |
| 5-Jul | 0.00% | 0.00% | 0.32% | 0.62% | 0.00% | 4.00% | -- | 0.00% | 0.00% | 0.53% |
| 12-Jul | -- | 0.00% | 0.37% | 0.60% | 0.00% | -- | -- | -- | 0.00% | 0.54% |
| 19-Jul | -- | -- | 0.86% | 0.86% | -- | 0.00% | -- | -- | 0.00% | 0.86% |
| 26-Jul | -- | -- | 1.10% | 0.88% | -- | 0.00% | -- | -- | 0.00% | 0.91% |
| 2-Aug | -- | -- | 2.38% | 0.94% | -- | -- | -- | -- | -- | 1.07% |
| 9-Aug | -- | -- | 0.93% | 0.70% | 16.67% | 0.00% | -- | -- | -- | 0.76% |
| 16-Aug | -- | 0.00% | 1.81% | 0.74% | 0.00% | 0.00% | -- | -- | 0.00% | 0.81% |
| 23-Aug | -- | -- | 0.57% | 0.46% | 0.00% | -- | -- | -- | -- | 0.46% |
| 30-Aug | -- | 0.00% | 0.00% | 0.85% | -- | 100.00% | -- | -- | 0.00% | 0.86% |
| 6-Sep | -- | -- | 0.00% | 0.87% | -- | 0.00% | -- | 100.00% | -- | 1.09% |
| 13-Sep | -- | -- | 0.00% | 1.40% | -- | -- | -- | -- | -- | 1.34% |
| 20-Sep | -- | -- | 0.00% | 1.18% | -- | 0.00% | -- | 100.00% | 0.00% | 1.67% |
| 27-Sep | -- | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | -- | -- | -- | 0.00% |
| 4-Oct | -- | 0.00% | 0.00% | 0.66% | -- | 0.00% | -- | 20.00% | -- | 1.18% |
| 11-Oct | -- | 0.00% | 1.45% | 0.65% | -- | 0.00% | -- | 0.00% | 0.00% | 0.68% |
| 18-Oct | -- | -- | 0.00% | 0.41% | 0.00% | 0.00% | -- | 33.33% | -- | 0.47% |
| 25-Oct | -- | -- | 0.00% | 0.39% | -- | 0.00% | -- | 0.00% | -- | 0.36% |
| 31-Oct | -- | -- | 0.00% | 0.34% | -- | 0.00% | -- | 0.00% | 0.00% | 0.32% |
| # morts | 727 | 220 | 340 | 786 | 145 | 47 | 87 | 58 | 60 | 2,470 |
| # collected | 2,342,198 | 698,954 | 208,584 | 329,159 | 1,877,057 | 645,906 | 165,786 | 22,959 | 182,829 | 6,473,432 |
| % mortality | 0.03% | 0.03% | 0.16% | 0.24% | 0.01% | 0.01% | 0.05% | 0.25% | 0.03% | 0.04% |

Sample mortalities include dead fish removed from the sample tank prior to sampling and those from the sorting trough in the sample lab.  Annual sample mortality for all groups combined was 0.58% in 2018 (Table 14) and totaled 396 fish.  This is the lowest sample mortality rate on record since 1996.  The number of sample mortalities and mortality rate by species group was: 43 clipped (0.32%) and 15 unclipped (0.31%) yearling Chinook, 55 clipped (0.85%) and 227 unclipped (0.87%) subyearling Chinook, 35 clipped (0.30%) and 10 unclipped (0.27%) steelhead, 6 clipped (0.72%) and 5 unclipped (1.99%) sockeye/kokanee, and 0 Coho.  All species groups sample mortality rates were lower than the 2014 to 2017 average.  Sample mortality for all groups combined has ranged from a high of 1.05% in 2014 to a low of 0.58% in 2018.

Table 14.  Annual sample mortality by species group in percent at LWG, 2014-2018.

|  | Yearling Chinook |  | Subyearling Chinook |  | Steelhead |  | Sockeye/Kokanee |  | Coho |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | All | Total |
| 2014 | 0.61 | 0.68 | 1.38 | 1.41 | 0.40 | 0.46 | 0.00 | 4.06 | 0.35 | 1.05 |
| 2015 | 0.40 | 0.52 | 0.7 | 1.13 | 0.48 | 0.23 | 4.17 | 4.35 | 0.31 | 0.86 |
| 2016 | 0.38 | 0.36 | 0.95 | 1.18 | 0.23 | 0.16 | 4.57 | 6.25 | 0.12 | 0.74 |
| 2017 | 0.62 | 0.56 | 1.58 | 1.81 | 0.29 | 0.35 | 1.37 | 5.00 | 0.71 | 0.86 |
| 2018 | 0.32 | 0.31 | 0.85 | 0.87 | 0.30 | 0.27 | 0.72 | 1.99 | 0.00 | 0.58 |
| 14-17 ave | 0.50 | 0.53 | 1.18 | 1.30 | 0.33 | 0.31 | 3.04 | 4.26 | 0.37 | 0.88 |

Barge mortalities are salmonids removed from barge holds after the barges depart LWG.  The total number of smolts barged in 2018 included: 5,112,569 fish from LWG, 3,665,319 from LGS, and 2,306,744 fish from LMN.  The seasonal barge transport program mortality rate was 0.05% (5,394 of 11,084,632) (Table 16).  Barge mortalities by species group included:  2,945 clipped and 782 unclipped yearling Chinook, 270 clipped and 311 unclipped subyearling Chinook, 690 clipped and 283 unclipped steelhead, 50 clipped and 47 unclipped sockeye/kokanee, 15 Coho, and 1 fish not identifiable at release (Table 15).

Table 15.  Total annual transport program barge mortalities 2014-2018.

|  | Yearling Chinook |  | Subyearling Chinook |  | Steelhead |  | Sockeye/Kokanee |  | Coho | Unknown |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | All | Others | Total |
| 2014 | 1,562 | 427 | 217 | 404 | 476 | 159 | 4 | 328 | 100 | 0 | 3,677 |
| 2015 | 563 | 247 | 719 | 1462 | 599 | 258 | 84 | 19 | 13 | 1,720 | 5,684 |
| 2016 | 591 | 198 | 596 | 343 | 323 | 134 | 338 | 4 | 51 | 30 | 2,608 |
| 2017 | 1,729 | 534 | 405 | 693 | 456 | 243 | 33 | 9 | 17 | 0 | 4,119 |
| 2018 | 2,945 | 782 | 270 | 311 | 690 | 283 | 50 | 47 | 15 | 1 | 5,394 |
| 14-17 Ave | 1,111 | 352 | 484 | 725 | 464 | 199 | 115 | 90 | 45 | 437 | 4,022 |

Lower Granite truck transport fish mortality rate in 2018 was 0.19% (25 of 13,110) (Table 16).  The truck mortality number and percent by species included:  1 unclipped (14.3%) yearling Chinook, 4 clipped (0.60%) and 13 unclipped (0.10%) subyearling Chinook, and 7 unclipped (17.5%) sockeye/kokanee.  All trips were made with the 300 gallon pickup truck mounted tank except October 9, 11, and 31 when fish densities exceeded the midi tank capacity and the 3500 gallon semi-truck was used.

Table 16.  Annual percent truck mortality at LWG, 2014-2018.

|  | Yearling Chinook |  | Subyearling Chinook |  | Steelhead |  | Sockeye/Kokanee |  | Coho |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | Clipped | No Clip | All | Total |
| 2014 | 0.00 | 0.00 | 0.00 | 0.18 | -- | 0.00 | 0.00 | 1.56 | 0.00 | 0.18 |
| 2015 | 0.00 | 0.00 | 0.69 | 0.14 | 0.00 | 0.00 | -- | 0.00 | 0.00 | 0.14 |
| 2016 | 0.00 | 0.00 | 0.38 | 0.07 | 0.00 | 0.00 | -- | 0.00 | 0.00 | 0.08 |
| 2017 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |
| 2018 | -- | 14.3 | 0.60 | 0.10 | -- | 0.00 | -- | 17.50 | 0.00 | 0.19 |
| 14-17 Ave. | 0.00 | 0.00 | 0.28 | 0.13 | 0.00 | 0.00 | 0.00 | 1.22 | 0.00 | 0.13 |

--no fish trucked

##### Gas Bubble Trauma Monitoring (PSMFC)

Juvenile salmonids were sampled for GBT from April 5 through June 7 in 2018.  PSMFC personnel examined up to 100 clipped and unclipped yearling Chinook and steelhead each week for evidence of bubbles in paired and unpaired fins, and in the eye, as per Fish Passage Center GBT protocols.  This season 948 fish were sampled for GBT: 413 clipped and 132 unclipped yearling Chinook and 306 clipped and 97 unclipped steelhead.  During GBT sampling 12 PIT-tagged smolts were handled, not examined and returned to the separator including: 4 clipped and 1 unclipped yearling Chinook, 4 clipped and 3 unclipped steelhead.  An additional 21 fish were handled and released into the separator including: 1 clipped and 1 unclipped yearling Chinook, 3 clipped subyearling Chinook,  2 clipped steelhead, 11 clipped sockeye, and 3 Coho.   Smolts examined for GBT prior to April 23 were released in the sample recovery tank and bypassed.  Smolts examined for GBT after April 23 were returned to the raceways and transported.  A total of 310 smolts were bypassed including 147 clipped yearling Chinook, 90 unclipped yearling Chinook, 57 clipped steelhead, and 16 unclipped steelhead.  A total of 671 smolts were transported including:  271 clipped and 44 unclipped yearling Chinook, 3 clipped subyearling Chinook, 255 clipped and 84 unclipped steelhead, 11 clipped sockeye, and 3 coho.  There were 5 fish observed with symptoms of GBT while 21 were observed with symptoms in 2017.

Research

Four agencies participated in seven research projects at LWG juvenile facility that impacted 530,487 smolts which is 8.2% of the total collection or 10.4% of the 2018 facility collection for transport.  The 2014-2017 average was 487,363 smolts handled as part of LWG research projects.  The 530,487 smolts taken from the collection included: 145,037 clipped and 34,102 unclipped yearling Chinook, 52,469 clipped and 56,552 unclipped subyearling Chinook, 175,752 clipped and 34,771 unclipped steelhead, 14,932 clipped and 1,746 unclipped sockeye/kokanee, and 15,746 coho.  Also included were 750 subyearling Chinook salmon mortalities (without fin clip information) for the U.S. Geological Survey (USGS) stable isotope study. Corps biological staff collected 610 clipped and 527 unclipped steelhead kelts from the LWG juvenile facility for the Nez Perce Tribe (NPT) and Columbia River Inter-Tribal Fisheries Commission (CRITFC) this season.

## National Marine Fisheries Service (NMFS)-Study to Compare the Adult Returns of In-river Migrating versus Barged Juvenile Anadromous Salmonids (Transportation Study)

LWG Corps biological technicians collected smolts in the east raceways for NMFS tagging April 11 to June 16.  Raceway flows, fish behavior, and mortalities were monitored by Corps biological staff 24 hours per day.  Corps biological technicians collected 471,825 smolts for NMFS that were handled in their marking trailers at LWG fish facility as part of the annual transportation study.  Of these 36,422 smolts were PIT tagged and transported including 12,047 unclipped yearling Chinook, 8,105 clipped steelhead, and 16,270 unclipped steelhead.  There were 435,297 smolts handled that were not selected for tagging.  All fish were held overnight in the east raceways prior to transport.  There were 106 smolt mortalities reported by NMFS including 59 clipped and 13 unclipped yearling Chinook, 8 clipped and 1 unclipped subyearling Chinook, 18 clipped and 4 unclipped steelhead, 1 clipped and 1 unclipped sockeye/kokanee, and 1 Coho.  There were an additional 346 mortalities removed by Corps biological staff from the east raceways while being used exclusively for holding NMFS research fish.  Unclipped yearling Chinook with fork lengths less than 124 mm were targeted.  Unclipped steelhead with fin erosion were not PIT tagged.

## National Marine Fisheries Service (NMFS)-Study to Estimate Juvenile Salmonid Reach Survival

Corps biological technicians collected smolts in the east raceways for the continuing in-river survival study April 11 to June 16 in conjunction with the NMFS Transportation Evaluation study.  NMFS handled 47,647 smolts as part of this study, 47,596 that were PIT-tagged and bypassed including 11,870 unclipped yearling Chinook, 19,714 clipped steelhead, and 16,012 unclipped steelhead.  There were 51 post tagging mortalities including 36 unclipped yearling Chinook, 8 clipped steelhead, and 7 unclipped steelhead.  There were 346 mortalities removed from the east raceways while used exclusively for NMFS as described above.

## National Marine Fisheries Service (NMFS)-Monitoring the Migrations of Wild Snake River Spring/Summer Chinook

This study to monitor the migration behavior and survival of Snake River Basin wild spring/summer Chinook salmon aims to characterize migration timing, estimate individual wild population parr-to-smolt survival to LWG, and influence of environmental factors on migration patterns.  Fish PIT-tagged in natal streams during the summer of 2017 were diverted to the SByC tanks at LWG April 5 to June 8 during 2018.  Corps biological technicians monitored tank flow, fish behavior, and for mortalities 24 hours/day.  A total of 704 fish were impacted by this study.  NMFS handled and bypassed 461 targeted unclipped yearling Chinook as part of this study.  An additional 39 untagged smolts and 204 non-target PIT-tagged fish were incidentally diverted to the SBC tanks handled and bypassed.

## Idaho Fish and Game (IDFG)-Genetic Stock Identification

The goal of this study is to develop detailed genetic profiles for natural origin salmon and steelhead, develop genetic stock identification (GSI) techniques to estimate stock-specific escapement over LWG, monitor abundance, productivity and distribution of naturally produced adult and juvenile steelhead and salmon and to research and monitor stock-specific life history characteristics.  The objective of the study is to enumerate and characterize the natural production of spring/summer Chinook salmon and steelhead above LWG with regards to age composition and genetic stock identification.  Lower Granite biological staff collected 3,005 fish that were sorted by SMP biologists and provided to IDFG for this study April 2 to June 29.  Scale samples and fin clips were taken from 1,063 non-fin eroded unclipped steelhead and fin clips only were taken from 1,942 non-coded wire tag (CWT) unclipped yearling Chinook.  Sampled fish for this study were bypassed prior to transport collection (April 2-April 23) and included 143 unclipped steelhead without fin erosion and 948 yearling Chinook without CWT.  After April 23 sampled fish for this study were transported.

## University of Idaho/Columbia River Intertribal Fisheries Commission (CRITFC)/Nez Perce Tribe (NPT)-Evaluate Reproductive Success of Natural-Origin, Hatchery-Origin, and Kelt Steelhead in the Columbia River Basin

Corps biological staff collected 1,137 steelhead kelts from the Lower Granite juvenile separator from April 2 to June 28.  The purpose of the study is to evaluate steelhead kelt physiology and endocrinology for rehabilitating.  NPT/CRITFC personnel took genetic samples, PIT-tagged, and returned to the tailrace 873 steelhead, including 607 clipped and 266 unclipped steelhead kelts collected at LWG that did not meet their criteria.  Of the kelts collected, 261 steelhead kelts were transported to Dworshak National Fish Hatchery for acclimation and feeding studies of which 3 were clipped steelhead kelt mortalities.

USGS-Subyearling Chinook Stable Isotopes

The results of this effort will be used to explore use of stable isotopes to distinguish hatchery from natural origin subyearling Chinook. Stable isotope information is collected by USGS personnel to determine signatures specific to hatchery and naturally produced subyearling Chinook and signatures specific to individual release hatcheries.  Corps biological staff collected 750 subyearling Chinook mortalities between May 20-July 12 as part of the LWG daily facility collection mortality from raceways and holding tanks. SMP personnel bagged and froze the collected mortalities that were later released to USGS for their project.

PNNL Post Construction Evaluation of the Upgraded Juvenile Bypass System at Lower Granite Dam).  This study evaluated travel time and fish condition of hatchery yearling Chinook through the upgraded components of the LWG juvenile bypass system.    PIT-tagged unclipped hatchery yearling Chinook raised at PNNL (Leavenworth stock) were released into gatewell slots 1A and 5A and passed through the 14-inch orifices that were fitted with PIT tag detection and recaptured in the sort by code (SBC) sample tank. Passage times were measured for various sections of the bypass system.  Fish collected in the SBC tanks were assessed for descaling and injuries. There were 714 fish released into gatewell slot 1A, 714 fish released into gatewell slot 5A, and 15 fish released directly into the separator.  There were 2 mortalities prior to tagging and 8 post tagging mortalities. A total of 1,443 fish were released with 1,067 recovered in SBC tank. The remaining 376 Leavenworth stock unclipped hatchery yearling Chinook were not recovered in the SBC tank at LWG and likely were missed by the sampling gate and returned to the river.. There were 79 non-target PIT-tagged fish recovered in the SBC tanks as bycatch.  They included 24 clipped yearling Chinook, 18 unclipped yearling Chinook, 28 clipped steelhead, and 9 unclipped steelhead. Another 1,291 non-PIT-tagged fish were also recovered in the SBC tanks during this study including 541 clipped and 193 unclipped yearling Chinook, 1 unclipped subyearling Chinook, 446 clipped steelhead, 108 unclipped steelhead.  There were 2 clipped yearling Chinook mortalities collected in the SBC tank.

PNNL System Survival Study

This study evaluated effects of increased spill on juvenile yearling chinook and juvenile steelhead in-river survival through the hydrosystem and evaluated passage survival at LWG.  Fish were provided from LWG daily condition sample. Fish were PIT-tagged and acoustic-tagged prior to being released above and below LWG.  Fish released at Blyton Landing (11.5 miles above LWG) included 344 clipped and 123 unclipped yearling Chinook, 502 clipped and 891 unclipped subyearling Chinook, 521 clipped and 159 unclipped steelhead.  Fish released into the LWG tailrace included 211 clipped and 88 unclipped yearling Chinook, 261 clipped and 429 unclipped subyearling Chinook, and 372 clipped and 128 unclipped steelhead.  Fish released at Central Ferry (13 miles above LGO) included 230 clipped and 70 unclipped yearling Chinook, 246 clipped and 444 unclipped subyearling Chinook, and 393 clipped and 107 unclipped steelhead. There were 213 fish collected and bypassed without being part of the study.  Excess fish collected for the study were bypassed prior to April 23 (213 fish) and included 52 clipped and 46 unclipped yearling Chinook, 12 clipped and 9 unclipped subyearling Chinook, and 78 clipped and 16 unclipped steelhead. Excess fish collected for the study after April 23 (765 fish) were transported and included 185 clipped and 54 unclipped yearling Chinook, 123 clipped and 174 unclipped subyearling Chinook, and 155 clipped and 74 unclipped steelhead. There were 40 mortalities prior to tagging.  Pre-tagging mortalities included 1 clipped yearling Chinook, 10 clipped and 13 unclipped subyearling Chinook, and 10 clipped and 6 unclipped steelhead. There were 19 mortalities after tagging and before release. Post tagging mortalities included 5 clipped yearling Chinook, 3 clipped and 7 unclipped subyearling Chinook, and 1 clipped and 3 unclipped steelhead.

## Operation and Maintenance

##### Turbine Operations

Efforts were made to operate all turbine units within one percent of the peak efficiency from April 1 to October 31.  Deviations were infrequent and brief or required by BPA.  Table 17 contains unit outages during 2018.

Table 17.  Lower Granite turbine unit outages, 2018.

| Unit | Date OOS | Reason out of service |
| --- | --- | --- |
| Units 1 – 6 | February 27 – March 2 | Trash rack raking |
| Units 1 – 6 |  | ESBS installation |
| Units 1, 3 – 6 | March 1 - 3 | RAS Testing |
| Units 1 - 5 | August 13 - 16 | Doble Testing |
| Units 2 - 6 | November 13 - 15 | ESBS Removal |
| Unit 1 | January 1 - 12 | Blade Runner Repair |
|  | January 13 - 29 | Unit 1 Commissioning Testing |
|  | February 12 - 16 | Distribution Valve Removal |
|  | March 5 - 29 | Servo Oil Supply Repair |
|  | March 19 | ESBS Installation |
|  | April 5 | Gate Lock Oil Leak |
|  | April 30 | Governor Oil Pump |
|  | November 13 - 26 | Thrust Bearing Electrical Isolation |
|  | December 27 - | Digital Governor Upgrade/Annual Maintenance |
| Unit 2 | March 29 – April 3 | ESBS Installation (Forced Outage Delay in Installation) |
|  | August 27 - September 5 | EAL Grease Replacement |
| Unit 3 | March 29 | ESBS Installation |
|  | June 18 - 27 | Governor Oil Leak Repair |
|  | July 7 | 3C Headgate Position Verification |
|  | July 14 - 16 | Oil Leak 3C Intake Gatewell Slot |
|  | October 1 – November 8 | Digital Governor Upgrade/Annual Maintenance |
|  | November 12 - 13 | Loss of Communication for AGC Control. |
| Unit 4 | March 29 – April 2 | ESBS Installation (Forced Outage Delay in Installation) |
|  | September 20 | Stilling Basin Survey |
|  | November 7 – December 20 | Digital Governor Upgrade/Annual Maintenance |
| Unit 5 | January 22 - 25 | TO2 PM |
|  | March 29 | ESBS Installation |
|  | August 20 – September 28 | Digital Governor Upgrade/Annual Maintenance |
| Unit 6 | February 5 | Governor Prep Measurement |
|  | March 29 – April 2 | ESBS Installation (Forced Outage Delay in Installation) |
|  | July 9 – August 24 | Digital Governor Upgrade/Annual Maintenance |
|  | September 10 - 21 | OPTP, PLC Module Upgrade |

##### Debris/Trash Racks

Trashracks were raked the February 27 through March 2.  Trashrack raking was not required during the fish passage season.

# Extended-length Submersible Bar Screens (ESBSs)

ESBSs were inspected and tested prior to installation.  Screens were installed in unit 1 prior to returning the unit to service.  ESBSs were installed in units 3 and 5 March 29.  The remaining ESBSs were installed in units 2, 4, and 6 on April 2 and 3. Delays in ESBSs were due to commissioning and tie in of Phase 1a.  ESBSs were removed November 13-15 due to juvenile bypass upgrade programming and constructions repairs.  Brush cleaning cycle was set to operate every two hours this season.

##### Vertical Barrier Screens (VBSs)

VBSs were video inspected in conjunction with ESBSs during the 2018 fish passage season.  Detailed inspections were performed during the June ESBS inspection.  VBS screen panel mesh has the potential to deteriorate and become brittle over time.  VBS panels for screens that pass underwater camera inspection but showed potential for deterioration continue to be replaced/repaired during unit annual outages or during winter maintenance as time permits.

##### Gatewells

Gatewells were normally less than 1% covered with debris and did not exceed the 50% debris surface coverage criterion.  Turbulence in gatewells with ESBSs causes debris to tumble around and exit through the orifices rather than accumulate on the gatewell surfaces.  Surface debris was removed from individual gatewells with a hand dipping basket during initial water-up in late March and continued throughout the season.  Occasional oil sheens were dealt with by floating oil absorbent pads in the affected gatewells.

##### Orifices/Collection Channel

During the 2018 season the number of open orifices remained consistent with all 18 of the 14 inch orifices open.  Orifices were inspected every three hours and cycled and back-flushed with air to remove debris as needed March 29-August 15.  Irregular 14” orifice flow was observed in slots 2A and 4A May 4.  Both 14” orifices were closed and three 10” orifices were opened.  Mortar splatter from construction in the collection channel was removed from the 14” orifices and operation was returned to all 14” orifices that day.  Orifice operation programming continues as part of Phase 1a.  The facility was operated by two biological technician to monitor the orifice gallery and the operation of the newly completed PDW during the 2018 season.  Orifice lights were checked daily.

##### Primary Dewaterer

Lower Granite’s historic primary dewatering structure was replaced with the Phase 1a JBS upgrade primary dewaterer (PDW) and bypass system.  Primary dewaterer floor screen brushes, side screen brushes, and the pneumatic screen cleaners were operated in manual mode by powerhouse operators due to mechanical and programing issues with the new system.  All overflow weirs required manual operation until May 9 when weirs in groups A. B, and C were returned to auto and group D continues to be in manual for optimal PDW flow to the transport flume.  Programming, mechanical, and structural issues with the PDW will be addressed during the ongoing Phase 1a construction.

##### Wet Separator/Distribution and Sampling Systems

Water levels in the separator varied with the forebay elevation and PDW operations requiring adjustment in porosity control valves and separator exit gates.  Adjustments in flume flow were made to reduce fish holding in the transport flume and under the separator.  Porosity control valves were adjusted to prevent water from drying out on the east side of the porosity plate.  This operation resulted in increased volume of water exiting the separator.  Modifications to the porosity control unit to balance water across the plate and enable adequate dewatering will be made as time permits during the winter maintenance season.

Biological technicians adjusted porosity dewatering valves and exit gate positions in response to separator water elevation changes related to PDW weir operation. Separator exit gates were adjusted due to increased volume from the separator exits to improve PIT tag detection efficiencies per PSMFC request.

##### Barge Loading Operations

Barge loading operations occurred from April 24 through August 14.  Loading from the raceways went smoothly this season.  Direct loading did not occur.

##### Truck Loading Operations

Truck transport occurred from August 16 through October 31.

Recommendations

- Complete Phase 1a modifications and programming.

- Operate the PDW flume outflow between 35-40 cfs to reduce delays in system.

- Modify porosity control system operating valves.

- Replace porosity control system round perforated plate with narrower oval perforation plate.

- Replace mesh tailscreens with porosity plates to allow lamprey passage.

- Improve sample recovery truck loading pipe slope to eliminate fish stranding in pipe.

- Continue rebuilding motors on the 2000 series barges.

- Replace barge bumper cable and tire system with actual bumpers.

- Paint hulls on 8000 series barges.

- Install ballast material in barges 4394 and 4382 voids to eliminate use of river water.

- Upgrade trucks and tanks and purchase a trap and transport service truck.

- Install electronic operators for raceway supply knife gate valves.

- Improve juvenile collection facility pneumatic system air compressors and air lines.

- Replace sample holding tank fish exit release manual valves with pneumatic valves.

- Improve/modify anesthetic chamber door operation.

- Permanently close the collection channel 5A research weir that is becoming a safety concern.

- Modify the JFF sample anesthetic system and procedure to minimize volume of MS-222 waste and develop filter and disposal systems for LWG JFF MS-222.

- Ensure all researcher working at LGW are accountable for anesthetic waste disposal in compliance with the EPA Clean Water Act.

APPENDIX TABLES

Appendix Table 1.  Daily collection and bypass numbers and river conditions at Lower Granite Dam, 2018.

Appendix Table 2.  Percent descaling and daily facility mortality numbers at Lower Granite Dam, 2018.

Appendix Table 3.  Daily number of fish trucked and barged from Lower Granite Dam, 2018.

Appendix Table 4.  Daily number of adult fallbacks and fallback mortality at Lower Granite Dam, 2018.